�������������������������������������������������������������Ŀ

�                  P R E - V E R S I O N                      �

���������������������������������������������������������������

M A N U A L

M O D E M 6  /  C A L L - M O D E M

by

JanSoft International Corp. (Switzerland)

Jan SAGGIORI

5, chemin de la chevillarde

1208 GENEVE (Switerland)

Tel : (41-22) 348.77.55   Fax : (41-22) 349.86.74

Fidonet : 2:301/327

Telecommuncation program under Ms-Dos

Quickly also under Ms-Windows from Microsoft !

"Now, made easy telecommunication !"

�������������������������������������������������������������Ŀ

�                �  �Ŀ ڿ� �Ŀ �Ŀ �� ���                    �

�                �  �Ĵ �� �Ŀ � � ��  �                     �

�              ���  � � � � ��� ��� �   �   Vers.             �

���������������������������������������������������������������

********** INDEX - Manual MODEM6 / CALL-MODEM communication program ************

Chapiter 1 : First steps on MODEM6

1.1 Introduction

1.2 First steps on communication field

1.3 Main Menu on MODEM6 software. (Alt R - Option "Show Menu")

- Alt C : Automatic connection, or PhoneBook

- Alt S : Host Mode, Server, BBS (option)

- Alt H : Hang Up line

- Alt R : Show Menu

- Alt Z : Update informations

- Alt I : Init modem

- Alt X : Quit Call- Modem

- Alt O : Configuration

- Alt E : Dos External Service

- Alt U : Upload a file

- Alt D : Download a file

- Alt U : Upload a file

- Alt N : HAYES version Numb.

Chapiter 2 : First steps on MODEM6

2.1 Introduction

2.2 How to use Phonebook function keys ?

- Using "#/#  To choose"

- Using "C  Change data" or "Spc  Change data"

- Using "PgUp  Scroll up" or "PgDn  Scroll dn"

- Using "Home First page" or "End  Last page"

- Using "#� Dial number"

- Using "P  Print index"

- Using "M  Manual number"

- Using "D   Delete data"

- Using "Esc   End, exit"

Chapiter 3 : Upload programs to another system

3.1  Introduction

Chapiter 4 : Download programs from another system

4.1  Introduction

Chapiter 5 : HAYES Orders

5.1  Hayes command functions

5.1.1  General informations

5.1.2  How to configure via Registers

5.1.3  MICROCOM and MNP specific HAYES orders

5.2 Most known result codes

Chapiter 6 : How to configure your modem

6.1  Modems avalaible on SETUP.EXE : Configurations

Chapiter 7 : HAYES codes index

7.1  HAYES codes Index

Chapiter 8 : Tutor : How to connect to another system via modem

8.0  First steps

8.1  Communication - Introduction

8.2  Connections

8.3  Software control

8.3.1 Load and dial number from Phone Book

8.3.2 Results of Dialing order

8.3.3 Connection to choosed BBS : Phone number => 781 51 89

Chapiter 9 : Using SETUP program to configure MODEM6

9.1  First menu : Choosing modem type-configuration

9.2  Second menu : Configuring COM port, Baud rate, Parity, ...

Chapiter 10 : More informations about us and MODEM6 software

10.1  What's new about ?

******* Chapiter 1 - Manual MODEM6 / CALL-MODEM communication program **********

�������������������������������������������������������������Ŀ

�            Chapiter 1 : First steps on MODEM6               �

���������������������������������������������������������������

1.1 Introduction

------------

June 1993, Geneva - Switzerland. (Europe)

First, we thank you for using this software with the modem that you have,

we hope you will enjoy on communication field with your computer and modem.

1.2 First steps on communication field

----------------------------------

This is the first screen that you will see after calling MODEM6 program.

You can see the name of the author, his address, 100% Hayes compatibility

standard, date of release and version.

* With this screen you can know :

Version          : 1.16

Creation date    : 05.05.93

Serial port used : COM2 (Serial Port 2)

* Calling from Dos prompt this software you have 2 choices :

- If "in-line Order" to configure software

Syntax is :  "MODEM6.EXE Com_Port Speed u"

Where :

. Com_Port is  1, 2, 3 or 4

. Speed is 110, 300, 1200, 2400, 9600, 19200, 38400, 57600

. u is to have no port configuration

- If MODEM6.CFG configuration

Use "Configuration" on main menu to configure correctly your software,

with right serial port, data bit, parity, etc...

Example of screen :

-------------------

������������������������������������������������������������������������������Ŀ

�             CALL-MODEM Version 1.16     Revision date 05.05.93               �

�                Copyright all over the World by Jan SAGGIORI                  �

�                      5, Chemin Chevillarde 1208 GENEVA                       �

�                                                                              �

�        �������������������������������������������������������������Ŀ       �

�        �  JanSoft Copyright 1992   MODEM Orders call using the COM2  �       �

�        ���������������������������������������������������������������       �

�                                                                              �

�         This program works with all modems compatible HAYES.                 �

�         Syntax :  MODEM6.EXE Com_Port Speed u                                �

�                                                                              �

�            *  Com_Port is  1, 2, 3 or 4                                      �

�            *  Speed is 110, 300, 1200, 2400, 9600, 19200, 38400, 57600       �

�            *  u is to have no port configuration                             �

�                                                                              �

�        �������������������������������������������������������������Ŀ       �

�        �                �  �Ŀ ڿ� �Ŀ �Ŀ �� ���                    �       �

�        �                �  �Ĵ �� �Ŀ � � ��  �                     �       �

�        �              ���  � � � � ��� ��� �   �   Vers.             �       �

�        ���������������������������������������������������������������       �

�                                                                              �

�      Attention ! : The modem program uses the COM2, to quit press ESC        �

��������������������������������������������������������������������������������

** Wait a moment, Initialization **

******* Page 2/10 - Manual MODEM6 / CALL-MODEM communication program ***********

1.3 Main Menu on MODEM6 software. (Alt R - Option "Show Menu")

----------------------------------------------------------

Example of the menu :

---------------------

����������������������������������������������������������������������ͻ

�  Function Keys    CALL - MODEM    � COM2 : 38400 N 8 1 � Addr : 760  �

����������������������������������������������������������������������Ķ

�                                   �                                  �

�  Automatic connection :  Alt C    �   Configuration       :  Alt O   �

�                                   �                                  �

�  Host Mode (Server)   :  Alt S    �   Dos External Service:  Alt E   �

�                                   �                                  �

�  Hang up line         :  Alt H    �   Upload a file       :  Alt U   �

�                                   �                                  �

�  Show Menu            :  Alt R    �   Download a file     :  Alt D   �

�                                   �                                  �

�  Update Informations  :  Alt Z    ����������������������������������ĺ

�                                   �                                  �

�  Init modem           :  Alt I    �   ROM version Number  :  Alt V   �

�                                   �                                  �

�  Quit  Call-Modem     :  Alt X    �   HAYES version Numb. :  Alt N   �

�                                   �                                  �

����������������������������������������������������������������������ͼ

- Alt C : Automatic connection, or PhoneBook :

--------------------------------------------

With this option you will be able to access to PhoneBook to choose

phone number that you want to dial. See on PhoneBook section on Chap. 2

for more informations.

- Alt S : Host Mode, Server, BBS (option) :

-----------------------------------------

You can now using this function transform you computer system on a

server, host or BBS to have access from outside system to your computer.

- Alt H : Hang Up line :

----------------------

Pressing this combination of keys you can "Hang Up" line.

- Alt R : Show Menu :

-------------------

The Alt R key will clear all the screen, show you all main menu with

all function keys. Remember that the colors of letter will be only white

and background color will be blue.

- Alt Z : Update informations :

-----------------------------

Informations about this software, like Updates/modifications...

Example of the screen :

-----------------------

����������������������������������������������������������������������ͻ

� January 1993 �eta version,                    Next Version, February �

�                    ��������������������������Ŀ                      �

�                    �   UPDATE  INFORMATIONS   �                      �

�                    ����������������������������                      �

�                                                                      �

�  On next versions will be added to MODEM6 these new features :       �

�  * Protocols : Telink, Cis B, WXmodem, Modem7 (Checksum/CRC), Imodem �

�    Sealink, BImodem, Jmodem, etc.                                    �

�  * Terminals : VT52, VT100, TTY, Tektronix 4010, ADM3a, ADM5, TV925. �

�  * Complete Host Mode will be added.                                 �

��[Done]��������������������������������������������������������������ĺ

�  * Protocols : Ascii (Normal), Ascii (Raw) ready.              v1.12 �

�    Xmodem (Crc-Ch), Ymodem (Bat-1Kb), Ymodem-G, Kermit, Zmodem v1.14 �

�  * Cursor routine adjusted because bugs. (Totaly new)          v1.13 �

�  * Changed the general philosophy of func. All from main page. v1.13 �

�  * Screens saved after functions Update, Dwnld, Upld...        v1.13 �

�  * ANSI added to this software, and some more new functions.   v1.14 �

�  * New Speeds : To comply with modems 19200, 38400, 57600 bps. v1.15 �

�  * Phone-Book with number, baud rate, parity. (Can be saved !) v1.15 �

�  * Store configuration of Port, rate, parity, Data & Stop bit. v1.15 �

����������������������������������������������������������������������ͼ

- Alt I : Init modem :

--------------------

Using this option, you will configure again your modem attributes,

but this will be done with parameters that you gave during INSTALLATION

of the program. (With SETUP.EXE file)

Example of the screen :

-----------------------

**************************************

*                                    *

*          I Init the modem          *

*                                    *

*              W  A  I  T            *

*                                    *

**************************************

<ESC> To Continue

- Alt X : Quit Call- Modem :

--------------------------

You will exit to dos, and have screen shown below. This will end

execution of your program and terminate all communications.

Example of the screen :

-----------------------

������������������������������������������������������������������������������Ŀ

�  (C) Copyright 1992 by  JanSoft                                              �

�                                                        Version 1.16          �

�                                                                              �

�  Thanks for using Call-Modem !!!                                             �

��������������������������������������������������������������������������������

C:\MODEM6>_

- Alt O : Configuration :

-----------------------

You can reconfigure directly from the software all communication

parameters like BAUDS, DATA BITS, STOP BITS, PARITY.

* Press 1-2-3-4 for COM ports : from COM1 to COM4

* Press B to change Bauds     : from 110 to 57600

* Press D to change Data bits : 5/6/7/8

* Press S to change Stop bits : 1/2

* Press A to change Parity    : N/E/O/M/S

Pressing "Space" key, you can save the modem configuration that you

have just configured on the screen.

Example : "COM2:38400,N,8,1" if saved with configuration below.

Example of the screen :

-----------------------

����������������������������������������������������������������������ͻ

�  Function Keys    CALL - MODEM    � COM2 : 38400 N 8 1 � Addr : 760  �

����������������������������������������������������������������������Ķ

�                                   �                                  �

�  Automatic connection :  Alt C    �   Configuration       :  Alt O   �

�                                   �                                  �

�  Host Mode (Server)   :  Alt S    �   Dos External Service:  Alt E   �

�                                   �                                  �

�  Hook phone line      :  Alt H    �   Upload a file       :  Alt U   �

�                                   �                                  �

�  Show Menu            :  Alt R    �   Download a file     :  Alt D   �

�  ����������������������������������������������������������������������ͻ

�  �         Configuration             �      Copyright 1992 JanSoft      �

�  ����������������������������������������������������������������������Ķ

�  � show in    :  Ascii & Ansi        � Baud       :   38400 bps         �

�  � tx/rx      :  Automatic           � Data bits  :   8 bits  (5/6/7/8) �

�  � compatible :  Hayes 100 %         � Stop bits  :   1 bit(s)    (1/2) �

�  � duplex     :  Automatic           � pArity     :   N     (N/E/O/M/S) �

��ͺ Emulation  :  Ascii & Ansi        � port       :   COM2              �

����������������������������������������������������������������������ͼ

- Alt E : Dos External Service :

------------------------------

Using this option you will be able during communication with other

systems, to have access to dos. You will be free using all you want.

You are not obliged with this option to quit this software and stop

communication, and typing "EXIT" to dos prompt you will be immediatly

again under communication.

Example of the screen :

-----------------------

Type EXIT to return to Modem6...

Microsoft(R) MS-DOS(R) Version 6

(C)Copyright Microsoft Corp 1981-1993.

C:\MODEM6>_

- Alt U : Upload a file :

-----------------------

With this option you will be able sending by phone files to another

system using protocols listed below, for more informations see

Chapiter 3.

Example of the screen :

-----------------------

��� Upload �����������������������������������������������������������������Ŀ

�                ��   ��    �   ���   �    ��   �   �     ��                 �

�                � �  � �  � �   �   � �  �    � �  �    ��                  �

�                ��   ���  � �   �   � �  �    � �  �      �                 �

�                �    � �   �    �    �    ��   �   ���  ��                  �

�    a. Ascii                * O.K.          n. Sealink           * Disable  �

�    b. Ascii  (Raw)         * O.K.          o. WXmodem           * Disable  �

�    c. Xmodem (Checksum)    * O.K.          p. Zmodem (DSZ)      * O.K.     �

�    d. Xmodem (CRC)         * O.K.          q. Compuserve B      * Disable  �

�    e. Xmodem 1K            * O.K.          r. Compuserve B+     * Disable  �

�    f. Xmodem 1K-G          * O.K.          s. Imodem            * Disable  �

�    g. Ymodem   (& Batch)   * O.K.          t. BImodem           * Disable  �

�    h. Ymodem G (& Batch)   * O.K.          u. Jmodem            * Disable  �

�    i. Telink               * Disable       v. JanModem          * Disable  �

�    j. Kermit               * O.K.          w. External 1        * Disable  �

�    k. Super-Kermit         * O.K.          x. External 2        * Disable  �

�    l. Modem7 (Checksum)    * Disable       y. External 3        * Disable  �

�    m. Modem7 (CRC)         * Disable       z. External 4        * Disable  �

�                                                                            �

�������������������������������������������������������������� 26 Protocols ��

- Alt D : Download a file :

-------------------------

With this option you will be able receiving by phone files from

another system using protocols listed below, for more informations see

Chapiter 4.

Example of the screen :

-----------------------

��� Download ���������������������������������������������������������������Ŀ

�                ��   ��    �   ���   �    ��   �   �     ��                 �

�                � �  � �  � �   �   � �  �    � �  �    ��                  �

�                ��   ���  � �   �   � �  �    � �  �      �                 �

�                �    � �   �    �    �    ��   �   ���  ��                  �

�    a. Ascii                * O.K.          n. Sealink           * Disable  �

�    b. Ascii  (Raw)         * O.K.          o. WXmodem           * Disable  �

�    c. Xmodem (Checksum)    * O.K.          p. Zmodem (DSZ)      * O.K.     �

�    d. Xmodem (CRC)         * O.K.          q. Compuserve B      * Disable  �

�    e. Xmodem 1K            * O.K.          r. Compuserve B+     * Disable  �

�    f. Xmodem 1K-G          * O.K.          s. Imodem            * Disable  �

�    g. Ymodem   (& Batch)   * O.K.          t. BImodem           * Disable  �

�    h. Ymodem G (& Batch)   * O.K.          u. Jmodem            * Disable  �

�    i. Telink               * Disable       v. JanModem          * Disable  �

�    j. Kermit               * O.K.          w. External 1        * Disable  �

�    k. Super-Kermit         * O.K.          x. External 2        * Disable  �

�    l. Modem7 (Checksum)    * Disable       y. External 3        * Disable  �

�    m. Modem7 (CRC)         * Disable       z. External 4        * Disable  �

�                                                                            �

�������������������������������������������������������������� 26 Protocols ��

- Alt V : ROM version Number :

----------------------------

This option is for technical purpose only, in fact you will have no

interest knowing the ROM version of your Modem. This will only help you

if you have an incompatiblity problem or some bugs during execution of

order by your modem.

In case of bugs, you can contact your supplier and give him this

version number and HAYES version number to resolve problems quicker.

In example below, you will see that the version number is "163".

Example of the screen :

-----------------------

��������������������������������Ŀ

�   Version number of the ROM    �

����������������������������������

AT I1

163

OK

- Alt N : HAYES version Numb. :

-----------------------------

Like we explained you on "ROM version number", this option is for

technical purpose only.

In current example, you will see that the version number is "Ver M1.2".

Example of the screen :

-----------------------

������������������������������������Ŀ

�   "Hayes orders"  version Number   �

��������������������������������������

AT I0

Ver M1.2

OK

******* Chapiter 2 - Manual MODEM6 / CALL-MODEM communication program **********

�������������������������������������������������������������Ŀ

�            Chapiter 2 : First steps on MODEM6               �

���������������������������������������������������������������

2.1  Introduction

------------

This chapiter will be consecrated to "PhoneBook" service only. You will

learn how you can use current orders :

PgUp  Scroll up     Home First page     P  Print index

C  Change data      PgDn  Scroll dn     End  Last page

*M  Manual number   *D   Delete data     Esc   End, exit

#/#  To choose      #� Dial number      Spc  Change data

Example of the screen :

-----------------------

������������������������������������������������������������������������������ͻ

�   N�  Name                      NUMBER          BAUD   P D A D     ORDER     �

������������������������������������������������������������������������������Ķ

� # 1  Transnet (Gen�ve CH)     733 80 50          9600  N 8 1 F  ___________  �

�   2  Gonisoft (Node 1)        757 65 73         38400  N 8 1 F  ___________  �

�   3  Gonisoft (Node 2)        757 65 87         38400  N 8 1 F  ___________  �

�   4  Sun Server (Gen�ve)      300 35 80          9600  N 8 1 F  ___________  �

�   5  Sun Server (Gen�ve)      300 45 48          9600  N 8 1 F  ___________  �

�   6  CCTI (Gen�ve 9600)       781 51 89         38400  N 8 1 F  ___________  �

�   7  CCTI (Gen�ve 2400)       781 51 41         38400  N 8 1 F  ___________  �

�   8  SwissAir                 049 047 111C       9600  N 8 1 F  ___________  �

�   9  Central Point Software   001 503 6906650    2400  N 8 1 F  ___________  �

�   10 Wayne Bell (California)  001 213 2086689    9600  N 8 1 F  ___________  �

�   11 Tyffany (California)     001 510 4586404   38400  N 8 1 F  ___________  �

�   12 Omen (Usa)               001 503 6213746    9600  N 8 1 F  ___________  �

�   13 Deep Thought (Usa)       001 512 2441598    9600  N 8 1 F  ___________  �

�   14 Flushot + (Usa)          001 914 5862025    9600  N 8 1 F  ___________  �

������������������������������������������������������������������������������Ķ

� PgUp  Scroll up     Home First page     P  Print index       C  Change data  �

� PgDn  Scroll dn     End  Last page     *M  Manual number    Spc Change data  �

� Esc   End, exit     #/#  To choose      #� Dial number      *D  Delete data  �

������������������������������������������������������������������������������Ķ

� Message :                            � 38400 COM2 N 8 1 � Port Address : 760 �

������������������������������������������������������������������������������ͼ

2.2 How to use Phonebook function keys ?

------------------------------------

2.2.1  Using "#/#  To choose"

----------------------

Using #/# keys you will scroll one line or more of the screen, and you

will be able seeing other phone modem numbers.

2.2.2  Using "C  Change data" or "Spc  Change data"

--------------------------------------------

Pressing "C" or "Space", you will be able changing data on current line.

You will change fields "Name", "Number", "Baud rate", "Parity", "Data

bit", "Stop bit", "Duplex". Pressing "Enter" on a field, old data will

not be removed.

2.2.3  Using "PgUp  Scroll up" or "PgDn  Scroll dn"

--------------------------------------------

Pressing PgUp (or PgDn) you will scroll 14 lines below (if PgDn), and

see following page.

2.2.4  Using "Home First page" or "End  Last page"

-------------------------------------------

Pressing "Home" you will go directly to beggining of Phonebook, if you

press "End" you will directly jump to last number page of Phonebook.

2.2.5  Using "#� Dial number"

----------------------

Pressing "#�" you will order to system to dialing current phone number

and to connect system.

a.  More about DIALING

The  Model 2400 originates a call by dialing a remote modem, using

either  pulse or DTMF (dual-tone multifrequency) dialing depending

on  the dial subcommand issued by the user. The tandem mode can be

used  in call progress dialing to switch between pulse and tone in

the middle of a dial sequence.

Dialing can be performed in one of two possible modes:

1)  Call  progress mode where the modem monitors the  progress  of

the call, or

2)  Blind mode where the modem does not monitor the call  progress

signal after dialing is completed.

When  a  dial  command  is entered, the  modem  places  itself  in

originate  mode  unless  reverse  mode dialing  is  specified.  If

reverse  mode dialing is specified, the modem reconfigures  itself

in the answer mode when dialing is completed.

If  call  progress  mode  is enabled, the modem  will  detect  the

presence  of  a  dial tone before proceeding with  the  dialing  o

numbers. When call progress mode is disabled, the modem uses blind

dialing  in  which the numbers are dialed without waiting for  the

presence  of a dial tone. The modem is also capable of  overriding

its  dialer,  thus  allowing  manual  dialing  using  an  external

telephone  handset.  After dialing, the modem waits for an  answer

tone from the remote modem.

a.1  CALL PROGRESS DIALING

Once  the dialing is entered, the modem places itself off hook and

looks  for the dial tone indicate signal. It assumes tone  dialing

(unless pulse dialing is requested) and dials the entered number.

If  a "W" is encountered while numbers are being dialed, the modem

searches  for another dial tone indicate signal before  proceeding

with  the remainder of the dialing sequence. When the dialing  se-

quence  is complete, the modem searches for a remote ring  signal.

If this signal is detected, it them searches for an answer carrier

tone. The modem waits for an originate carrier tone if the reverse

mode is selected (see "R" command).

When  the  carrier  tone is detected, the modem is placed  in  the

originate  mode  and completes the modem handshake  sequence.  The

modem is then automatically placed in the data mode.

a.2  BLIND DIALING

Blind  dialing is similar to call progress dialng except that  the

call  progress signals are disabled and ignored by the modem. Once

offhook,  the  modem  delays  the dialing  sequence  by  the  time

specified  in status register S5 (wait for dial tone). If a "+" is

encountered  in the dialing sequence, it is ignored. To wait for a

second dial tone (for PBX systems), a comma "," (pause subcommand)

must be inserted between the numbers to be dialed. After the dial-

ing  sequence is complete, the modem searches for an answer  tone.

The  remainder  of the sequence is identical to the call  progress

dialing modem handshake sequence, described in Section 6.1.

a.3  AUTO ANSWER MODE

When   the  auto-answer  configuration  is  selected,  the   modem

automatically  answers the phone after "n" rings. The parameter  n

is  set using the command "Sr=n". The value in status register  S0

controls  the  number of rings that must occur  before  answering.

After  the  telephone  is answered, the modem  waits  two  seconds

(billing  delay) during which the modem sends only an answer  car-

rier  tone  to the originating modem, and then proceeds  with  the

call  establishment  handshake  as  specified  by  the  respective

protocol.  After  the  handshake is completed, the modem  sends  a

result  code  to  the host (unless disabled) and enters  the  data

mode.

If the "A" command is used and status register S0 is set to 0, the

modem answers the phone without wating for a ring.

a.4  DATA CALL TERMINATION

Connection may be teminated by either the host or the remote modem

in the following ways:

Data  Terminal Ready.....When the Data Terminal Ready (DTR) signal

goes  low  for more than 50 ms, the modem begins a disconnect  se-

quence  (except for certain test modes). The modem then returns to

the command mode.

Carrier  Loss.....If the carrier signal is lost for more than  300

ms, the modem returns to the command mode. The telephone line con-

nection is automatically broken and the modem placed onhook if the

command mode is re-entered as a result of a carrier loss.

Abort.....If the carrier signal is not detected within a specified

time  interval  after  dialing,  the modem goes  onhook  and  then

returns to the command mode.

a.5  HANDSHAKE PROTOCOL

Modem handshake protocols for Bell 103J, Bell 212A, V.22, and V.22

bis   are   specified   by   the   respective   Bell   and   CCITT

recommendations.  After  the handshake sequence is  complete,  the

modem  automatically enters the data mode. An exception occurs  if

the modem has been given the command to return to the command mode

after dialing.

2.2.6  Using "P  Print index"

----------------------

Pressing "P" you will order to print complete Phonebook on printer,

naturally verify if printer is connected.

2.2.7  Using "M  Manual number"

------------------------

Not in use. Available on next version.

2.2.8  Using "D   Delete data"

-----------------------

Not in use. Available on next version.

2.2.9  Using "Esc   End, exit"

-----------------------

Pressing "Esc", you will abord Phonebook function and return to main

page.

******* Chapiter 3 - Manual MODEM6 / CALL-MODEM communication program **********

�������������������������������������������������������������Ŀ

�       Chapiter 3 : Upload programs to another system        �

���������������������������������������������������������������

3.1  Introduction

------------

��� Upload �����������������������������������������������������������������Ŀ

�                ��   ��    �   ���   �    ��   �   �     ��                 �

�                � �  � �  � �   �   � �  �    � �  �    ��                  �

�                ��   ���  � �   �   � �  �    � �  �      �                 �

�                �    � �   �    �    �    ��   �   ���  ��                  �

�    a. Ascii                * O.K.          n. Sealink           * Disable  �

�    b. Ascii  (Raw)         * O.K.          o. WXmodem           * Disable  �

�    c. Xmodem (Checksum)    * O.K.          p. Zmodem (DSZ)      * O.K.     �

�    d. Xmodem (CRC)         * O.K.          q. Compuserve B      * Disable  �

�    e. Xmodem 1K            * O.K.          r. Compuserve B+     * Disable  �

�    f. Xmodem 1K-G          * O.K.          s. Imodem            * Disable  �

�    g. Ymodem   (& Batch)   * O.K.          t. BImodem           * Disable  �

�    h. Ymodem G (& Batch)   * O.K.          u. Jmodem            * Disable  �

�    i. Telink               * Disable       v. JanModem          * Disable  �

�    j. Kermit               * O.K.          w. External 1        * Disable  �

�    k. Super-Kermit         * O.K.          x. External 2        * Disable  �

�    l. Modem7 (Checksum)    * Disable       y. External 3        * Disable  �

�    m. Modem7 (CRC)         * Disable       z. External 4        * Disable  �

�                                                                            �

�������������������������������������������������������������� 26 Protocols ��

******* Chapiter 4 - Manual MODEM6 / CALL-MODEM communication program **********

���������������������������������������������������������������Ŀ

�      Chapiter 4 : Download programs from another system       �

�����������������������������������������������������������������

4.1  Introduction

------------

��� Download ���������������������������������������������������������������Ŀ

�                ��   ��    �   ���   �    ��   �   �     ��                 �

�                � �  � �  � �   �   � �  �    � �  �    ��                  �

�                ��   ���  � �   �   � �  �    � �  �      �                 �

�                �    � �   �    �    �    ��   �   ���  ��                  �

�    a. Ascii                * O.K.          n. Sealink           * Disable  �

�    b. Ascii  (Raw)         * O.K.          o. WXmodem           * Disable  �

�    c. Xmodem (Checksum)    * O.K.          p. Zmodem (DSZ)      * O.K.     �

�    d. Xmodem (CRC)         * O.K.          q. Compuserve B      * Disable  �

�    e. Xmodem 1K            * O.K.          r. Compuserve B+     * Disable  �

�    f. Xmodem 1K-G          * O.K.          s. Imodem            * Disable  �

�    g. Ymodem   (& Batch)   * O.K.          t. BImodem           * Disable  �

�    h. Ymodem G (& Batch)   * O.K.          u. Jmodem            * Disable  �

�    i. Telink               * Disable       v. JanModem          * Disable  �

�    j. Kermit               * O.K.          w. External 1        * Disable  �

�    k. Super-Kermit         * O.K.          x. External 2        * Disable  �

�    l. Modem7 (Checksum)    * Disable       y. External 3        * Disable  �

�    m. Modem7 (CRC)         * Disable       z. External 4        * Disable  �

�                                                                            �

�������������������������������������������������������������� 26 Protocols ��

******* Chapiter 5 - Manual MODEM6 / CALL-MODEM communication program **********

�������������������������������������������������������������Ŀ

�                 Chapiter 5 : HAYES Orders                   �

���������������������������������������������������������������

5.1  Hayes command functions

-----------------------

5.1.1 General informations :

----------------------

This is general information which applies to ALL more knowns modems.

Other files may provide additional detail on specific modem models.

A.  MODEM COMMAND SET

Direct  communication  with the modem is performed using the  "AT"

command  set.  The modem accepts commands from the host when it is

in the command mode only. To issue a command while the modem is in

the data mode, it must be placed in the command mode by issuing an

escape code sequence.

MODEL 2400 "AT" COMMANDS

COMMAND             DESCRIPTION

AT                  COMMAND LINE PREFIX (ATTENTION CODE);

PRECEDES COMMAND LINES EXCEPT +++ (ESCAPE)

AND A/ (REPEAT) COMMANDS.

A/                 RE-EXECUTE LAST COMMAND LINE; (A/ IS NOT

FOLLOWED BY A CARRIAGE RETURN).

A                  OFF-HOOK IN ANSWER MODE.

B                  SELECTS CCITT V.22 OPERATION WHEN

COMMUNICATING AT 1200 BPS.

B1  (Default)      SELECTS BELL 212A OPERATION WHEN

COMMUNICATING AT 1200 BPS.

D                  DIAL NUMBER WHICH FOLLOWWD IN THE COMMAND

LINE.

E                  MODEM DOES NOT "ECHO" COMMANDS BACK TO

TERMINAL.

E1  (Default)      MODEM SECHOS COMMANDS BACK TO TERMINAL.

H  (Default)       ON-HOOK (HANG-UP) COMMAND.

H1                 GO OFF-HOOK.

I                  REQUEST PRODUCT IDENTIFICATION CODE.

I1                 PERFORMS CHECKSUM ON FIRMWARE ROM; RETURNS

CHECKSUM.

I2                 PERFORMS CHECKSUM ON FIRMWARE ROM; RETURNS

OK OR ERROR.

M                  SPEAKER OFF.

M1  (Default)      SPEAKER ON UNTIL CARRIER DETECTED.

M2                 SPEAKER ALWAYS ON.

M3                 SPEAKER ON UNTIL CARRIER DETECTED EXCEPT

DURING DIALING.

O                  RETURN TO ON-LINE STATE.

O1                 RETURN TO ON-LINE STATE AND INITIATE RETRAIN.

Q  (Default)       MODEM RETURNS RESULT CODES.

Q1                 MODEM DOES NOT RETURN RESULT CODES.

SRT=N              SET REGISTER R TO VALUE N.

SR?                REQUEST CONTENTS OF REGISTER R.

V                  SHORT FORM NUMERIC RESULT CODES.

V1  (Default)      FULL WORD RESULT CODES.

X                  300 BPS COMPATIBILITY MODE; CONNECT RESULT

CODE ENABLED.

X1                 MODEM BLIND DIALS; ALL CONNECT XXXX RESULT

CODES ENABLED; BUSY SIGNAL NOT DETECTED.

X2                 MODEM WAITS FOR DIAL TONE BEFORE DIALING;

ALL CONNECT XXXX RESULT CODES ENABLED; MODEM

SEND BUSY RESULT CODE IF BUSY DETECTED.

X3                 MODEM BLIND DIALS; ALL CONNECT XXXX RESULT

CODES ENABLED; MODEM SENDS BUSY CODE IF BUSY

DETECTED.

X4  (Default)      MODEM WAITS FOR DIAL TONE BEFORE DIALING;

ALL CONNECT XXXX RESULT CODES ENABLED; BUSY

SIGNAL DETECTED.

Z                  LOAD STORED CONFIGURATION PROFILE.

&C                 DCD ALWAYS ON.

&C1  (Default)     DCD ON INDICATES PRESENCE OF DATA CARRIER.

&D   (Default)     MODEM IGNORES DTR.

&D1                MODEM ASSUMES COMMAND STATE WHEN ON-TO-OFF

TRANSITION DETECTED ON DTR.

&D2                MODEM HANGS UP & ASSUMES COMMAND STATE.

AUTO ANSWER NOT DISABLED UNLESS EXTERNALLY

IMPLEMENTED BY (RI- # DTR-)= A6-.

&D3                MODEM ASSUMES INITIALIZATION STATE UPON

DETECTING AN ON-TO-OFF TRANSITION ON DTR.

&F                 LOAD FACTORY CONFIGURATION PROFILE.

&P  (Default)      PULSE DIAL MAKE/BREAK RATIO = 39/61 (UK/HK).

&P1                PULSE DIAL MAKE/BREAK RATIO = 33/67 (UK/HK).

&S                 DSR ALWAYS ON.

&S1                DSR OPERATES IN ACCORDANCE WITH EIA RS-232-C

SPEC.

&T                 TERMINATE TEST PROGRESS.

&T1                INITIATE LOCAL ANALOG LOOPBACK TEST.

&T8                INITIATE LOCAL ANALOG LOOPBACK WITH SELF TEST.

&Z                 STORE TELEPHONE NUMBER.

MODEL 2400 "AT" COMMAND DEFINITIONS

A     Go off hook and answer phone immediately. The Model 2400

immediately goes off hook, transmits the answer tone and

waits for a carrier from the remote mode.

Commands entered after the A command are ignored.

A/    Repeat previous command. The last command executed by the

Model 2400 will execute again. This command does not require

the "AT" prefix or <cr> termination.

Bn    Bell/CCITT mode select.

B0 - Selects CCITT V. 22 operation at 1200 bps.

B1 - Selects BELL212A operation at 1200 bps (Default).

Most 1200 bps modems in the United States operate using the

BELL212A specification. In order to communicate with a modem

conforming to the international CCITT V.22 standard, issue

the B0 command before dialing.

Dn    Dial the number immediately following this command where

n = digits 0 through 9 together with the dial modifiers

listed below.

The Model 2400 will dial the number specified in the manner

determined by the dial modifiers. The dial modifiers may

appear anywhere within the dial string with the exception

of the ; modifier which must appear at the end of the

string.

Dial Modifiers:  P  Pulse dial

T  Tone dial

R  Originate call in answer mode

W  Wait for dial tone before dialing

,  Pause before dialing next digit

;  Return to command mode after dialing

@  Wait for quiet answer before dialing

!  Flash-hook

S  Dial the number stores in RAM

EXAMPLE:  ATDP9WT713,SS;

In this example the Model 2400 pulse dials a 9 digit, waits

for a dial tone, them tome dials the digits 713. Next, the

modem pauses the length of time specified in register S8. It

them dials the number stored in the Model 2400 controller's

RAM which can be up to 32 digits in length. Finally, the

modem returns to the command mode after the final digit has

been dialed.

The P modifier causes the modem to pulse or rotary dial at a

fixed rate of 10 pulses per second. (The make/break ratio

for pulse dialing is determined by the &P command.)

DIAL MODIFIERS

P                   PULSE DIAL

T                   TONE DIAL

,                   PAUSE

!                   FLASH

@                   WAIT FOR SILENCE

W                   WAIT FOR SECOND DIAL TONE

;                   RETURN TO COMMAND STATE AFTER DIALING

R                   ORIGINATE CALL IN ANSWER MODE

D                   DIAL STORED NUMBER

COMMAND SUMMARY

AT        ATTENTION COMMAND

A/        REPEAT COMMAND

A         ANSWER MODE COMMAND

B/B0      SELECT CCITT V.22 IN 1200 BPS

B1        SELECT BELL 212A

Dn        DIAL COMMAND (n = DIAL STRING, 0-9, *, #, !, W, R, AND,

E         DISABLE ECHO

E1        ENABLE ECHO

H         ON HOOK (HANG UP)

H1        OFF HOOK

I/I0      IDENTIFY

I1        DISPLAY 000

I2        DISPLAY OK

I3        DISPLAY MESSAGE PROGRAM ROM

L/L0      LOW VOLUME

L1        LOW VOLUME

L2        MEDIUM VOLUME

L3        MAXIMUM SPEAKER VOLUME

M/M0      DISABLE SPEAKER

M1        SPEAKER ON UNTIL CARRIER

M2        SPEAKER ALWAYS ON

M3        DISABLE SPEAKER WHEN DIALING OR CARRIER

O         GO ON LINE

O1        RETRAIN (2400 BPS MODE ONLY)

Q/Q0      ENABLE RESULT CODES

Q1        DISABLE RESULT CODES

Sn=X      S REGISTER WRITE (n = VALID S REG, X = ALLOWED VALUE SEE

&F COMMAND)

Sn?       READ SPECIFIED S REGISTER (n = ANY VALID S REGISTER)

V0        SELECT DIGIT RESULT CODE

V1        SELECT WORD RESULT CODE

X0        BLIND DIAL, DISPLAY "CONNECT"

X1        BLIND DIAL, DISPLAY "CONNECT 1200/2400"

X2        DETECT DIAL TONE, DISPLAY "CONNECT 1200/2400"

X3        DETECT BUSY TONE, DISPLAY "CONNECT 1200/2400"

X4        DIAL AND BUSY TONE DETECT, DISPLAY "CONNECT 1200/2400"

Y0        DISABLE LONG SPACE DISCONNECT

Y1        ENABLE LONG SPACE DISCONNECT

Z         SOFTWARE RESET

+++       ESCAPE CODE (GO TO COMMAND MODE IF ON LINE)

&C0       DCD ALWAYS ON

&C1       DCD FOLLOWS REMOTE CARRIER STATUS

&D0       IGNORE DTR

&D1       GO TO COMMAND WHEN DTR ON TO OFF

&D2       HANGUP AND GO TO COMMAND WHEN DTR ON TO OFF

&D3       POWER UP RESET WHEN DTR ON TO OFF

&F        FETCH FACTORY CONFIGURATION for S REGESTERS

&G0       NO GUARD TONE

&G1       550 HZ GUARD TONE

&G2       1800 HZ GUARD TONE

&J0       SELECT RJ-11/RJ-41S/RJ-45S

&J1       SELECT RJ-12/RJ-13

&M0       SELECT ASYNCHRONOUS MODE

&P0       MAKE BREAK RATIO FOR US (40/60)

&P1       MAKE BREAK RATIO FOR FOREIGN STANDARD (33/66)

&R0       CTS TRACKS RTS

&R1       CTS ON WHEN MODEM READY IN DATA MODE

&S0       DSR ALWAYS ON

&S1       DSR FOLLOWS RE232 SPECIFICATION

&T0       STOP TEST

&T1       LOCAL ALB TEST

&T3       LOCAL DLB

&T        ENABLE DLB REQUEST

&T5       DISABLE DLB REQUEST

&T6       REMOTE DLB

&T7       REMOTE DLB AND SELF TEST

&T8       LOCAL ALB AND SELF TEST

&V        VIEW PROFILE

S-REG   ADDRESS  DEFAULT  VALID RANGE   DESCRIPTION

S0      228      0        0-255         RING TO ANSWER ON

S1      229      0        0-255         RING COUNT

S2      230      43       0-127         ESCAPE CODE CHARACTER

S3      231      13       0-127         CR CHARACTER

S4      232      10       0-127         LF CHARACTER

S5      233      8        0-127         BS CHARACTER

S6      234      2        0-255         WAIT FOR DIAL TONE

S7      235      30       1-99          WAIT FOR DATA CARRIER

S8      236      2        0-255         PAUSE TIME FOR COMMA

S9      237      6        1-255(.1SEC)  CARRIER DET RESP. TIME

S10     238      14       1-255(.1SEC)  LOST CARRIER TO HANG UP

DELAY

S11     239                             TOUCH TONE DURATION

S12     240      50       0-255         ESCAPE CODE GUARD TIME

(1/50SEC)

S13     241                             NOT USED

S14     242      AAH                    BIT MAPPED OPTION REG

S15     243                             NOT USED

S16     244      0                      MODEM TEST OPTIONS

S17     245                             NOT USED

S18     246      0        0-255         TEST TIMER

S19     247                             NOT USED

S20     248                             NOT USED

S21     249      0                      BIT MAPPED OPTIONS REG

S22     250      76                     BIT MAPPED OPTIONS REG

S23     251      7                      BIT MAPPED OPTIONS REG

S24     252                             NOT USED

S25     253      5        0-255(.01SEC) DELAY TO DTR (SYNC ONLY)

S26     254      1        0-255(.01SEC) RTS TO CTS DELAY (HALF

DUP)

S27     255      40H                    BIT MAPPED OPTION REG

5.1.2  How to configure via Registers

------------------------------

Here is a list of the S-registers.

ALL S REGISTER COMMANDS ARE ENTERED IN THE AT COMMAND LINE.

EXAMPLE: AT S8=2 S2=43 S7=30

TO READ THE VALUE IN ANY REGISTER USE

AT SX? <CR>

THE MODEM SHOULD ECHO THE VALUE FOUND IN THE REGISTER X

REG  Values      Register function

S0   0           AUTO ANSWER OFF

1-255       AUTO ANSWER ON, ANSWER ON INDICATED # OF RINGS

S1 **0-255       RING COUNT (THIS IS A READ-ONLY REGISTER)

S2   0-127       ESCAPE CHARACTER  NORMALLY SET TO 43 OR "+"

127-255     ESCAPE FUNCTION DISABLED

S3  0-12        7        END OF LINE CHARACTER  NORMALLY SET TO 13 (CR)

S4  0-127        LINE FEED CHARACTER  NORMALLY SET TO 10 (LF)

S5  0-127        BACKSPACE CHARACTER  NORMALLY SET TO 8 (BS)

S6  2-255  SEC   PAUSE BEFORE DIALING (NORMALLY 2 SEC)

S7  1-255  SEC   WAIT FOR CARRIER (NORMALLY SET TO 30 SEC)

S8  0-255  SEC   PAUSE FOR COMMA IN DIAL STRING (NORMALLY SET TO 2 SEC)

S9  1-255  .1SEC CARRIER VALIDATION TIME NORMALLY 6 (.6 SEC)

S10 1-255  .1SEC LOSS OF CARRIER DISCONNECT DELAY NORMALLY 14 (1.4 SEC)

S11 50-127 .1SEC TOUCH TONE DURATION. THE DEFAULT IS 90.

S12 20-255 .02SE ESCAPE SEQENCE PAUSE  NORMALLY 50 (1.0 SEC)

S13 *            POWER UP ASYNC DATA FORMAT (USE CHART BELOW)

S14 *            OPTION STATUS DEFULT AA (HEX CODE) (USE CHART BELOW)

S16 *            DEFULT 0. TEST STATUS (USE CHART BELOW)

S18 0-255 SEC    DEFULT 0. TEST TIMER (REMOTE TEST)

S21 *            DEFULT 0.OPTION STATUS  (USE CHART BELOW)

S22 *            OPTION STATUS, DEFULT 76. (USE CHART BELOW)

S23 *            OPTION STATUS  NORMALLY 07 (USE CHART BELOW)

S25 0-255 SEC    DTR DETECT DELAY NORMALLY 5 SEC

ASYNC MODE, SYIC AUTO-DIAL AND SYIC MANUAL DIAL MODE .05 SEC

S26 0-255 MS     RTS TO CTS DELAY (1 MS)

S27 *            OPTION STATUS, DEFULT 40 (HEX CODE)  (USE CHART BELOW)

* REGISTERS USED BY MODEM PROCESSOR. DO NOT ALTER THESE

** READ ONLY REGISTERS

S13 DATA FORMAT:

POSITION  VALUE  FUNCTION

BIT 0      -    NOT USED

BIT 1      -    NOT USED

BIT 2      0    PARITY OFF

1    PARITY ON

BIT 3      0    ODD PARITY

1    EVEN PARITY

BIT 4      0    7 DATA BITS

1    8 DATA BITS

BIT 5      0    BIT 8=0

1    BIT 8=1

BIT 6      -    NOT USED

BIT 7      -    NOT USED

S14 DATA FORMAT:

BIT 0      -    NOT USED

BIT 1      0    LOCAL CHARACTER ECHO OFF

1    LOCAL CHARACTER ECHO ON

BIT 2      0    STATUS DISPLAYS ON

1    STATUS DISPLAYS OFF

BIT 3      0    STATUS CODES SENT

1    STATUS MESSAGES SENT

BIT 4      0    MODEM RESPONDS TO COMMANDS

1    MODEM IGNORES COMMANDS

BIT 5      0    TONE-DIALING

1    PULSE-DIALING

BIT 6      -    NOT USED

BIT 7      0    ANSWER MODE

1    ORIGINATE MODE

S16 DATA FORMAT:

BIT 0      0    ANALOG LOOP OFF

1    ANALOG LOOP ACTIVE

BIT 1      -    NOT USED

BIT 2      0    DIGITAL LOOP OFF

1    DIGITAL LOOP ACTIVE

BIT 3      0    REMOTE DIGITAL LOOP REQUESTED BY OTHER MODEM IS OFF

1    REMOTE DIGITAL LOOP REQUESTED BY OTHER MODEM IS ACTIVE

BIT 4      0    REMOTE DIGITAL LOOP OFF

1    REMOTE DIGITAL LOOP ACTIVE

BIT 5      0    SELF TEST RDL OFF

1    SELF TEST RDL ACTIVE

BIT 6      0    SELF TEST ANALOG LOOP OFF

1    SELF TEST ANALOG LOOP ACTIVE

BIT 7      -    NOT USED

S21 DATA FORMAT:

BIT 0      0    RJ11/RJ41/45 JACK TYPE

1    RJ12/FJ13 JACK TYPE

BIT 1      -    NOT USED

BIT 2      0    CTS FOLLOWS RTS

1    CTS FORCED ON

BIT 3&4    0    MODEM IGNORES DTR

1    MODEM ASSUMES COMMAND STATE WHEN DTR TURNS OFF

2    MODEM ASSUMES COMMAND STATE AND DISABLES AUTO-ANSWER

WHEN DTR TURNS OFF

3    MODEM RESETS WHEN DTR TURNS OFF

BIT 5      0    DCD FORCED ON

1    DCD FOLLOWS CARRIER

BIT 6      0    DSR FORCED ON

1    DSR BEHAVES NORMALLY

BIT 7      0    LONG SPACE DISCONNECT DISABLED

1    LONG SPACE DISCONNECT ENABLED

S22 DATA FORMAT:

BIT 0&1    1    LOW SPEAKER VOLUME

2    MEDIUM SPEAKER VOLUME

3    HIGH SPEAKER VOLUME

BIT 2&3    0    SPEAKER ALWAYS OFF

1    SPEAKER ON UNTIL CARRIER DETECTED

2    SPEAKER ALWAYS ON

3    SPEAKER OFF DURING DIALING THEN ON UNTIL CARRIER DETECTED

BIT 4&5&6  0    X COMMAND IN EFFECT

4    X 1 COMMAND IN EFFECT

5    X 2 COMMAND IN EFFECT

6    X 3 COMMAND IN EFFECT

7    X 4 COMMAND IN EFFECT

BIT 7      0    39/61 MAKE/BREAK RATIO (PULSE DIALING)

1    33/67 MAKE/BREAK RATIO

S23 DATA FORMAT:

BIT 0      0    REMOTE DIGITAL LOOP REQUEST DENIED

1    REMOTE DIGITAL LOOP REQUEST GRANTED

BIT 1&2    0    0-300 BPS

1    600 BPS

2    1200 BPS

3    2400 BPS

BIT 3      -    NOT USED

BIT 4&5    0    EVEN PARITY

1    PARITY BIT ALWAYS 0

2    ODD PARITY

3    PARITY BIT ALWAYS 1

BIT 6&7    0    NO GUARD TONES

1    550 HZ GUARD TONE

2    1800 HZ GUARD TONE

S27 DATA FORMAT:

BIT 0&1    0    ASYNCRONOUS MODE

1    ASYNC/SYNC MODE

2    SYNC AUTO-DIAL

3    SYNC MANUAL DIAL

BIT 2      0    PUBLIC PHONE LINE

1    LEASED LINE

BIT 3      -    NOT USED

BIT 4&5    0    INTERNAL CLOCK USED

1    EXTERNAL CLOCK USED

2    SLAVED CLOCK USED

BIT 6      0    CCITT V.22 PROTOCOL (1200 BPS)

1    BELL 212A (1200 BPS)

BIT 7      -    NOT USED

OTHER MODEM COMMANDS:

---------------------

AT       STANDARD LEAD IN SEQUENCE (ATTENTION)

ATA      FORCE ANSWER

ATD      DIAL (ATD ###-#####)

P     PULSE (ATDP ###-####)

T     TONE  (ATDT ###-####)

ATE1     ENABLE LOCAL CHARACTER ECHO

ATE0     DISABLE LOCAL CHARACTER ECHO

ATH1     FORCE LINE CONNECT

ATH0     FORCE LINE DISCONNECT

ATI      ECHO VERSION TYPE

ATL1     SPEAKER VOLUME LOW

ATL2     SPEAKER VOLUME MEDIUM

ATL3     SPEAKER VOLUME HIGH

ATM0     SPEAKER ALWAYS OFF

ATM1     SPEAKER ON EXCEPT WHILE CARRIER PRESENT

ATM2     SPEAKER ALWAYS ON

ATM3     SPEAKER ON EXCEPT WHILE DIALING AND CARRIER PRESENT

ATO      GO ONLINE AFTER ESCAPE OR WHILE IN COMMAND MODE

ATQ0     STATUS DISPLAY ON

ATQ1     STATUS DISPLAY OFF

ATV0     STATUS MESSAGES SENT

ATV1     STATUS CODES SENT

ATX0     "CONNECT" (CODE 1)  ALL SPEEDS NO DIAL TONE DETECTION

ATX1     "CONNECT" (CODE 1)  0-300 BPS OR "CONNECT 1200" FOR 1200 OR

"CONNECT 2400" FOR 2400 BPS NO DIAL TONE DETECTION

ATX2     WAIT FOR DIAL TONE (CONNECT SPEEDS SHOWN AS FOR X1)

ATX3     DETECT BUSY SIGNAL (CONNECT SPEEDS SHOWS AS FOR X1)

ATX4     WAIT FOR DIAL TONE, DETECT BUSY SIGNAL (CONNECT SPEEDS SHOWS AS FOR X1)

ATZ      RESET TO POWER UP CONDITION (CLEARS ALL REGISTERS)

2400 MODEM SPECIAL COMMANDS:    (* = FACTORY SETTING STANDARD)

ATB0     CCITT V.22 AT 1200 BPS

ATB1     BELL 212A AT 1200 BPS *

AT &F    RESTORE TO FACTORY C0NFIGURATION

AT &G    NO GUARD TONE *

AT &G1   550 HZ GUARD TONE

AT &G2   1800 HZ GUARD TONE

AT &J    RJ11/RJ41S/RJ45 JACK *

AT &J2   RJ12/RJ13 JACK

AT &L    REGULAR PHONE LINE *

AT &L1   LEASED LINE

AT &M    ASYNC OPERATION *

AT &M1   ASYNC/SYNC OPERATION

AT &M2   SYNC AUTO DIAL

AT &M3   SYNC MANUAL DIAL

AT &P    39/61 PULSE MAKE/BREAK RATIO *

AT &P1   33/67 PULSE MAKE/BREAK RATIO

AT &T4   GRANT RDL TEST REQUEST *

AT &T5   DENY RDL TEST REQUEST

AT &V    VIEW STATUS OF ALL MODEM SETTINGS. SEE ALL S REGISTER SETTINGS AND

INTERNAL MODEM COMMAND SETTINGS ON ONE SCREEN!

AT &X    SYNC CLOCK INTERNAL *

AT &X1   SYNC CLOCK EXTERNAL

AT &X2   SYNC CLOCK SLAVED

AT &Z    STORE DIAL COMMAND LINE

DATA INTERFACE (2400)

AT &C    DCD ALWAYS ON

AT &C1   DCD ON WHILE CARRIER PRESENT *

AT &D    DTR IGNORED *

AT &D1   DTR FAIL DISCONNECT ENABLED (MODEM RETURNS TO COMMAND MODE)

AT &D2   DTR FAIL DISCONNECT ENABLED (MODEM RETURNS TO COMMAND MODE, TURN

AUTO ANSWER OFF)

AT &D3   DTR FAIL DISCONNECT ENABLED (RESET MODEM)

5.1.3  MICROCOM and MNP specific HAYES orders

--------------------------------------

The modem control commands can be divided in several categories:

I. STANDARD HAYES COMMANDS

A/      Repeat last command

A       Answer

Bn      CCITT/Bell 1200 bps operation

&Cn     DCD control

D       Dial

DS=n    Dial stored number

&Dn     DTR control in asynchronous mode

En      Command mode echo

&Fn     Restore factory defaults

&Gn     Select guard tone

Hn      Hook control

In      Identification

&Jn     Select jack

&Ln     Dial-up/Leased line

Ln      Speaker volume control

Mn      Speaker control

&M      Async/sync mode selection

On      Go back to on line state

P       pulse dial

&Pn     Dial pulse ratio

Qn      Disable result codes

&Rn     RTS/CTS control

Sn      Select S register

=       Set S register

?       Display S register

&Sn     DSR options

T       Tone dial

&Tn     Tests

Vn      short/long results codes

&V      show configuration

&Wn     Store configuration in EERAM

&Xn     Synchronous clock source

Xn      Call progress mode and connect messages

Zn      Recall EERAM configurations

&Zn=    Save phone number in EERAM

II. MICROCOM ADDITIONAL COMMANDS - GENERAL PURPOSE

\Bn     Transmit break to remote system

D/n     Dial stored number. Similar to DS=n. Added for MICROCOM

DL      Dial last number

%En     enable/disable auto retrain

\F      read stored phone numbers (subset of &V)

\Kn     Set break control

\Pn     Store phone number

%R      Display S registers

\S      Read modem status

%V      Display firmware version number (same as I3)

III. MICROCOM COMMANDS - MNP COMMANDS

\An     Maximum MNP block size

%An     Set Auto-Reliable Fallback character

\Cn     Set auto-reliable Buffer

%Cn     Enable/disable data compression (class 5)

\En     Set Data Echo in normal mode

\Gn     Set modem port flow control during NORMAL connection

\Jn     DTE interface Speed Adjust

\Nn     Set operating mode

\O      Originate reliable link

\Qn     Set DTE port flow control

\Tn     Inactivity Timer

\U      Accept reliable link

\Vn     Enable MNP result form

\Xn     Set XON/XOFF pass through

\Y      Switch to reliable

\Z      Switch to normal mode

All the commands in groups II and III, and some commands in group

I,  are documented here. It would be also useful to refer to  the

Microcom   "AX/1200  and  AX/2400  Series  Owner's  Manual"   for

additional details.

The  first  part  of the  documentation  specifies  each  command

individually.The  second  part contains  some  explanatory  notes

about  the  modem.The MNP S registers are specified in  the  last

section of the documentation.

DESCRIPTION OF THE COMMANDS

I. MODIFIED HAYES COMMANDS

&Fn     Recall factory configuration

Description

&F0     Recall Hayes factory default

&F1     Recall Microcom factory default

&F2     Recall CompuCom factory default (auto-reliable)

Action

if n = 0, recall configuration 0

if n = 1, recall configuration 1

if n = 2, recall configuration 2

if n > 2, say ERROR

Comments

&F0 is recalled at power-up. &F1 and &F2 is recalled only if  &F1

or &F2 commands are issued.

&Tn     Test

Description

&T0     stop test

&T1     local analog loop back

&T3     local digital loop back

&T4     enable response to digital loopback

&T5     disable response to digital loopback

&T6     initiate a remote digital loopback

&T7     RDLB with self test

&T8     local analog loop back with self test

any other parameter will return "ERROR"

&T1 and &T8 say ERROR if:

- not &M0

- already in test

- \N2 or \N3

&T3, &T6 and &T7 say ERROR if:

- not connected (therefore &M0 must be active for &T3/6/7 to work).

- already in test

- the connection is reliable

&M1, &M2, &M3 say ERROR if any test running (&T1/8/6/7/3).

&Wn     Store Current Configuration

Description

if n>=1, n=1. Store current configuration in EERAM conf. n.

Store registers:

S0, S14, S18, S21, S22, S23, S25, S26, S27 (hayes registers)

S36, S37, S38, S39, S40, S41 (MNP registers)

Stored parameters:

\A %A B \C &C &D E \E %E \G &G \J \K &L M &M \N P &P Q \Q &R T \T

&T4 &T5 V \V X \X S0 S18 S25 S26

Action Comments

Note that if the DTE speed is 110 bps, a speed of 300 bos is saved.

Y       Long space disconnect

Description

This  command was kept for compatibility with Hayes.  It  returns

"OK"  in  any  case but the command has an  effect  only  if  the

connection is direct.

ATY0 : ignore received long space

ATY1  : send a break for 4 seconds and hang-up if 1.6s  of  space

detected.

II. GENERAL PURPOSE ADDITIONAL COMMANDS

\Bn     Transmit break to remote system

Description

Send  a  scrambled  space to the  remote  system,  under  certain

conditions.

Action

* If not connected, say "NO CARRIER", do not send break.

* Else ...

D/n     Dial stored number.

Description

Dial  stored phone number specified by n. "n" must  be  comprised

between 0 and 3, otherwise "ERROR" is returned.

n  must  be  the last character in the  command  line,  otherwise

"ERROR" is returned.if n is omitted, n=0 is assumed

Action

(CMDBUF_PTR points to the character following "/")

If (CMDBUF_PTR = #BOTBUF+1) N=0;        /* end of buffer */

AL = [CMDBUF_PTR]

CMDBUF_PTR++;

If (AL = 0) N=0;                        /* end of line */

Else If (AL < '0' or AL > '3') goto ERROR;      /* invalid */

Else If (CMDBUF_PTR!=#BOTBUF+1 AND [CMDBUF_PTR]!=0) goto ERROR;

Else N=n

Display NV_STORED[n];

Dial NV_STORED[n];

Comments

This  command is not a standard Hayes command. It was  added  for

compatibility  with  MICROCOM.  It is similar to  DS=n,  but  not

identical:  in  particular  DS=n will  ignore  the  command  line

following  the  n, D/n will return "ERROR" if n is not  the  last

character. The ATDS= command is still valid.

Differences with MICROCOM implementation:

MICROCOM modems allow 10 phone numbers, numbered from 1 to 10.

We only allow 4 phone numbers, numbered from 0 to 3.

DL      Dial last number

Description

Dial  last number (displayed on 2nd line of AT\S command).  Since

the  last  dialed number is only saved in RAM, it  is  not  saved

between  power-up. This command was added for compatibility  with

Microcom.

The  "L" must be the 1st character after "D", and any  characters

following the L are ignored.

%En     enable/disable auto retrain

Description

%E0     disable auto-retrain

%E1      enable auto-retrain at 2400 bps when the signal  quality

is poor.

Action

If n>=1, n=1

S16.7 = n

Comments

S16.7 is checked in file DSP.ASM.

\F      read stored phone numbers

Description

If no ERAM present, send "NO NV MEM"

Else display the 4 phone numbers stored in ERAM.

No  parameter is needed. So AT\F0 will execute AT\F, then  return

ERROR because 0 is not a valid command

Comments

The code used is a subset of AT&V code. It was added for MICROCOM

compatibility

\Kn     Set Break Control

Description

Determines the type of action taken by the modem when a break  is

received from the DTE port or the modem port.

Action

If n>5, n=5

S40.7 6 5 = n

Comments

When breaks are received from the DTE or the modem line, or  when

a  "send  break"  command is issued, the  program  checks  the  S

register to take the appropriate action.

\Pn     Store phone number

Description

syntax:

AT..\P <CR>     equivalent to AT..\P0<CR>

AT..\Pn {optional string} <CR>

n  must be 0,1,2 or 3. Any other value following \P  will  return

"ERROR" (except for the AT..\P<CR> case, which assumes n=0).

The  valid  dial characters in the string are  stored  in  EERAM,

until the end of the command line, or 30 characters are stored.

Action

(CMDBUF_PTR points to the character following "\P")

If (CMDBUF_PTR=#BOTBUF+1) N=0   /* end of buffer */

AL = [CMDBUF_PTR]       /* read next char */

CMDBUF_PTR++            /* point to next */

if (AL=#0) N=0  /* end of line */

Else if (AL<'0' or AL>'3') ERROR        /* invalid parameter */

Else {  /* valid parameter */

N = AL - '0'

Store in NV_STORED[N]

}

Comments

This  command  is  Similar  to  Hayes &Z=  .  It  was  added  for

compatibility with Microcom, and the Hayes command AT&Z= is still

valid.

Unlike Microcom, only the valid dial characters in the string are

stored,  until 30 characters are stored. If the string  has  more

than 30 characters, the rest of the string is ignored.

Microcom allows phone numbers between 1 and 9, we allow between 0

and 3.

%R      Display Hayes S registers

Description

This  command  is provided for compatibility  with  Microcom.  It

displays registers S0 to S41 in hex and decimal format. The value

displayed  for  non existing registers (for example  S13)  is  0.

Example:

REG  DEC HEX    REG  DEC HEX    REG  DEC HEX    REG  DEC HEX

S00  000 00H    S01  000 00H    S02  043 2BH    S03  013 0DH

S04  010 0AH    S05  008 08H    S06  002 02H    S07  030 1EH

S08  002 02H    S09  006 06H    S10  014 0EH    S11  075 4BH

S12  050 32H    S13  000 00H    S14  170 AAH    S15  000 00H

S16  128 80H    S17  000 00H    S18  000 00H    S19  000 00H

S20  000 00H    S21  000 00H    S22  118 76H    S23  059 3BH

S24  000 00H    S25  005 05H    S26  001 01H    S27  064 40H

S28  000 00H    S29  000 00H    S30  003 03H    S31  054 36H

S32  020 14H    S33  157 9DH    S34  000 00H    S35  000 00H

S36  001 01H    S37  000 00H    S38  000 00H    S39  000 00H

S40  000 00H    S41  000 00H

Comment

No  parameter  is  read,  so AT%R0  will  read  %R,  display  the

registers, then read 0 and send an "ERROR" message.

\S      Read modem status

Description

Display current setting of the modem.

Action

We  don't  get  rid of eventual parameter  (so  \S0  will  return

"ERROR").  The  display  is identical to  the  Microcom  display,

except  for the options not implemented yet. The current  display

is as follows:

DIRECT  000:05:12

LAST DIAL       (408)2639300

MODEM BPS       2400    AT

MODEM MODE      DIR     AT\N1

MODEM FLOW      NONE    AT\G0

AUTO ANS.       OFF     ATS0=000

DTE BPS 2400    AT

BPS ADJUST      ON      AT\J1

DTE FLOW        OFF     AT\Q0

PASS XON/XOFF   OFF     AT\X0

PARITY  8N      AT

BREAK   5       AT\K5

EXIT CHAR       043     ATS2=043

COMMAND ECHO    ON      ATE1

RESULTS ON      ATQ0

RESULT TYPE     LONG    ATV1\V0

DATA ECHO       OFF     AT\E0

INACT TIMER     000     AT\T000

AUTO RETRAIN    ON      AT%E1

COMPRESSION     OFF     AT%C0

MAX BLK SIZE    64      AT\A0

AUTO BUFF       0       AT\C0

AUTO CHAR       000     AT%A000

PAUSE TIME      002     ATS8=002

DTR     1       AT&D1

CARR DET        1       AT&C1

SPEAKER CTRL    1       ATM1

LEASE LINE      OFF     AT&L0

ASYNC/SYNC      0       AT&M0

CTS/RTS ON      AT&R0

RDLB ENABLE     ON      AT&T4

DIAL MODE       3       ATX3

PULSE DIAL      US      AT&P0

GUARD TONE      0       AT&G0

BELL    OFF     ATB0

Comments

The  LAST  DIAL number is saved in RAM (not in EERAM), so  it  is

cleared when the power is turned off.

The  first  line  displays  the type  of  connection:  IDLE  (not

connection),  DIRECT,  NORMAL, RELIABLE  MNP4  (data  compression

disabled), RELIABLE MNP5 (data compression enabled).

The  time  displayed on the first line is  the  time  (HHH:MM:SS)

elapsed  since  the beginning of the current connection  (if  the

modem is on line), or the duration of the last connection (if the

modem is off line).

Note  that  if the S registers are set manually to  some  invalid

value  (example, S23.7,6=11), the \S display will show the  error

(i.e. GUARD TONE   3   AT&G3).

%V      Display firmware version number

Description

This  command  was  added for  compatibility  with  Microcom.  It

displays  the  same  message  as  I3  command.  No  parameter  is

expected, AT%V0 will display the identification message, then say

"ERROR".

III. MNP COMMANDS

\An     Maximum MNP block size

Description

\A0     block size max =  64 characters

\A1     block size max = 128 characters

\A2     block size max = 192 characters

\A3     block size max = 256 characters

\An, n>3 have the same effect as \A3.

Action

If n >= 3, assume n=3

S40.1,0 = n

%An     Set Auto-Reliable Fallback character

Description

Set  the  character  recognized as the  auto-reliable  fall  back

character  when  in answer mode and when option \C2  is  selected

(see \C command).

If n>=128, then 0 is assumed.

Action

If n>=128, n=0

S37 = n

\Cn     Set auto-reliable Buffer

Description

Determines  how  the  modem handles the data  received  from  the

remote modem, during the time interval in which it is  attempting

to  establish a reliable connection and only if it is  in  ANSWER

mode.  This option only takes effect at the end of  the  training

period  when  &M0  (async  mode)  and  \N3  (auto-reliable)   are

selected.

\C0   data  is  not buffered. if the remote modem is  a  non  MNP

modem,  any data sent by it during the 4 second wait period  will

be lost.

\C1  data buffered. If 200 characters buffered, switch to  normal

mode  and  pass  data  to the DTE. If  SYN  detected,  switch  to

reliable mode.

\C2   data  not buffered. If character defined  by  %A  received,

switch to normal mode, pass the character to the DTE

Action

n>2 => n=2

S38.1,0 = n

Comments

This  is  valid only if the modem is in  ANSWER  mode.  Originate

modems  ignore the \Cn setting and data is never buffered  during

the MNP handshake period.

%Cn     Enable/Disable Class 5

Description

%C0     Disable MNP5

%C1     Enable MNP5

Action

If n>1, n=1

S41.0 = n

Comments

If  MNP5  EQU  0 (in DEFINE.ASM), the code  is  assembled  for  a

SC11013  chip and MNP5 is not allowed. In that case, %C1  command

will return "ERROR" and data compression will always be disabled,

even if S41.0 is set "manually".

\En     Set Data Echo in normal mode

Description

\E0     Don't echo data sent by local DTE in normal data mode

\E1     Echo data sent by local DTE in normal data mode

Only valid if data mode is normal. Other data modes disable data

echo.

Action

n>2 => n=1

S38.3 = n

Comments

valid only in NORMAL mode

\Gn     Set modem port flow control during NORMAL connection

Description

Sets the type of flow control used to pace the data received from

the  remote modem (only in NORMAL mode. In direct mode,  no  flow

control  is  used.  In reliable mode, a special  scheme  of  flow

control is used and \Gn setting is ignored).

\G0     no flow control

\G1     XON/XOFF flow control

Action

if n>1 => n=1

Set S36.2 = n

\Jn     DTE interface Speed Adjust

Description

\J0     Don't change the speed of the serial port.

\J1     Change the speed of the serial port to match the bps rate

of the connection.

\Jn, n>1 behaves as \J1

Action

if n>1, set n=1

S38.2 = n

Comment

This  option is examined after the end of the MNP handshake:  the

"connect ..." message is always sent at the previous speed.  Then

if the speed of the connection is different from the speed of the

serial  port  and if \J1 is selected, the speed of  the  UART  is

changed.

In direct mode (\N1 selected), this option is ignored, it  always

behaves as \J1.

\Nn     Set operating mode

Description

Control  the type of connection when asynchronous mode  (&M0)  is

selected.

\N0     normal mode (non MNP, buffered)

\N1     do not use on SPEEDMODEM

\N2     reliable mode (MNP)

\N3     auto-reliable mode (MNP if possible, otherwise, normal)

Action

If n >= 3, assume n=3

S36.1,0 = n

Comments

Do  not  use during an active connection.  This  command  doesn't

affect the setting of &M command (S27.1,0):

After dialing, if &M0 is selected (S27.10=00), then the \N option

will  take effect, and the modem will eventually attempt  an  MNP

connection.  If  an asynchronous mode is selected, \N  option  is

ignored, and a synchronous connection (non MNP) is made.

Also, the MNP light is only turned on when a reliable  connection

is  established.  It  is not turned on as the  result  of  a  \Nn

command.

\O      Originate Reliable Link

Description

Initiate a reliable link, when the modem is in a direct or normal

connection.  If  the remote modem doesn't respond, go  to  normal

mode  (the remote modem must issue a AT\U command to establish  a

reliable connection).

*  If  the command was issued when not connected, NO  CARRIER  is

returned.

* If the modem was already in reliable mode, when AT\O is issued,

it goes back on line, immediately (without waiting for a response

from the remote modem).

* The rest of the command line (following \O) is ignored.

\Qn     Set DTE port flow control

Description

\Q0     disable DTE flow control

\Q1     XON/XOFF bidirectional flow control

\Q2     CTS unidirectional flow control

\Q3     CTS/RTS bidirectional flow control

When \Q1 is selected, the modem can tell the DTE to stop  sending

data by sending an XOFF character to the DTE. The modem will send

an XON character to tell the DTE to resume transmission. The  DTE

will tell the modem to stop sending data to the DTE by sending  a

XOFF  character,  and  to resume sending data by  sending  a  XON

character.

When \Q2 is selected, the modem uses CTS line to control the data

coming  from the DTE. The DTE has no way to control the  flow  of

data coming from the modem.

When \Q3 is selected, the modem uses CTS line to control the data

coming from the DTE, The DTE uses RTS line to control the flow of

data coming from the modem has no way to control the flow of data

coming from the modem.

(See flow control documentation)

Action

If n>=3, n=3

S36.7,6 = n

Comments

Simple  command.  This  command can be sent  when  the  modem  is

online,  by  returning  to  the  command  mode  using  an  escape

sequence. The new option will take effect as soon as returning to

data mode (using ATO for example).

\Tn     Inactivity Timer

Description

Defines  the number of minutes the modem waits before  hanging-up

when  no  data  is  sent  or  received  in  normal  or   reliable

connections.

\T0     disable the inactivity timer

\Tn     n = number of minutes. 0<n<=90

Action

If n>=90, n=90 (limit n to 90)

S39 = n

Comments

The  MNP  program is in charge of the counter and of  taking  the

appropriate action.

\U      Accept Reliable Link

Description

Accept  a reliable link, when the modem is in a direct or  normal

connection.  If  the remote modem doesn't respond, go  to  normal

mode  (the remote modem must issue a AT\O command to establish  a

reliable connection).

*  If  the command was issued when not connected, NO  CARRIER  is

returned.

* If the modem was already in reliable mode, when AT\U is issued,

it goes back on line immediately (without waiting for a  response

from the remote modem).

* The rest of the command line (following \U) is ignored.

\Vn     Enable MNP result form

Description

\V0     use standard Hayes result codes

\V1     use Microcom connect messages, in normal or MNP connect.

MNP connection

DTE speed       Long form               Short form

300             CONNECT 0300/REL        20

1200            CONNECT 1200/REL        22

2400            CONNECT 2400/REL        23

4800            CONNECT 4800/REL        24

9600            CONNECT 9600/REL        25

normal connection

DTE speed       Long form               Short form

300             CONNECT                 1

1200            CONNECT 1200            5

2400            CONNECT 2400            10

4800            CONNECT 4800            11

9600            CONNECT 9600            12

(The  DTE speed here, is after eventual BPS adjust, depending  on

\Jn  option.  See  additional  documentation  below  about  speed

issues.)

Action

if n>=1, n=1

S38.4 = n

Comments

This option is checked by CON_M_MRQ routine in MNPIOCS.ASM.

\Xn     Set XON/XOFF pass through

Description

This command takes effect in normal and reliable connections.

\X0      If  XON/XOFF  flow control  is  disabled,  the  XON/XOFF

characters  are  always passed through as regular  data.  If  \Q1

(XON/XOFF  DTE  flow  control) is  enabled,  XON/XOFF  characters

received  from the DTE are not sent to the remote system. If  \G1

(modem  flow  control) is selected, XON/XOFF  received  from  the

remote system is not sent to the local DTE.

\X1      XON/XOFF  characters are always passed through  in  both

directions  (from  local  DTE to remote system  and  from  remote

system to local DTE). Action is taken if flow control is  enabled

(\Q  for DTE flow control or \G for line flow control  in  normal

mode).

Any parameter bigger than 1 produces the same effect as \X1.

(See flow control documentation for more details)

Action

if n>=1, n=1

S36.3 = n

Comments

This  command can be sent when the modem is online, by  returning

to the command mode using an escape sequence. The new option will

take effect when going back to data mode (using ATO for  example,

or at next connection).

\Y      Switch to reliable mode

Description

Initiate (if the modem dialed) or accept (if the modem  answered)

a  reliable  link,  when  the modem is  in  a  direct  or  normal

connection.  If  the remote modem doesn't respond, go  to  normal

mode  (the remote modem must issue a AT\Y command to establish  a

reliable connection).

*  If  the command was issued when not connected, NO  CARRIER  is

returned.

* If the modem was already in reliable mode, when AT\Y is issued,

it goes back on line immediately (without waiting for a  response

from the remote modem).

* The rest of the command line (following \Y) is ignored.

\Z      Go from reliable to normal mode

Description

*  If  the command was issued when not connected, NO  CARRIER  is

returned.

* If the modem was in direct mode, it stays in direct mode.

* If the modem was in normal mode, it stays in normal mode.

* The rest of the command line (following \Z) is ignored.

NOTES

1. FLOW CONTROL

DO NOT USE XON/XOFF FLOW CONTROL WITH THE SPEEDMODEM USE ONLY

HARDWARE FLOW CONTROL,  RTS/CTS.

The  flow control settings (\Gn, \Xn, \Qn) can be changed  during

an active connection by escaping to command mode (using +++), and

issuing  the desired command. The new flow control settings  will

take place after returning in data mode (using ATO).

\Xn COMMAND

The  \Xn  option  is used to decide what happens  with  the  flow

control characters when flow control is enabled.

When \X0 is selected:

* DTE flow control: if DTE flow control is not selected, XON/XOFF

received  from  the  local DTE are treated as  regular  data  and

therefore  sent  to  the remote modem. If  DTE  flow  control  is

selected  (  &M0  async mode + Normal  (\N0)  or  reliable  (\N2)

connection  + \Q1 option ) when receiving XON/XOFF  flow  control

characters  from the local DTE, the modem takes  the  appropriate

action (stop or resume sending data to the DTE), but the XON/XOFF

characters are not sent to the remote modem.

* Modem port flow control: if modem flow control is not selected,

XON/XOFF  received from the remote system are treated as  regular

data  and therefore sent to the local DTE. If modem flow  control

is  selected  ( &M0 async mode + Normal (\N0)  connection  +  \G1

option ) when receiving XON/XOFF flow control characters from the

remote  system, the modem takes the appropriate action  (stop  or

resume  sending  data  to the remote system),  but  the  XON/XOFF

characters are not sent to the local modem.

When \X1 is selected:

* The XON/XOFF characters received from the local DTE are sent to

the remote system.

Then  if XON/XOFF DTE flow control is enabled ( &M0 async mode  +

Normal  (\N0)  or reliable (\N2) connection + \Q1  option  )  the

modem  stops sending characters to the DTE after  receiving  XOFF

from  DTE,  and  resumes  sending characters  to  the  DTE  after

receiving XON from the DTE.

*  The  XON/XOFF characters received from the remote  system  are

sent to the local DTE as regular data.

Then, if modem XON/XOFF flow control is enabled ( &M0 async  mode

+  Normal \N0 connection + \G1), the modem stops sending data  to

the remote system when XOFF is received over the phone line,  and

resumes sending data when XON is received over the phone line.

In conclusion, the way \Xn works is shown in the table below  (we

suppose a normal or reliable connection ).

XON/XOFF received from local DTE

\Q0             : pass to remote system through phone line

\Q1     \X0     : do not pass to remote system

\X1     : pass

XON/XOFF received from the remote system over the phone line

\G0             : pass to local DTE

\G1     \X0     : do not pass

\X1     : pass to local DTE

When \X1 is selected, XON/XOFF characters are always passed through.

When \X0 is selected, XON/XOFF characters are not passed  through

if  the  corresponding flow control is enabled; they  are  passed

through  if  the  corresponding flow control  is  disabled.  (see

figure).  Note that \G option has no effect on characters  coming

from local DTE; \Q option has no effect on characters coming from

the remote system.

NOTE:  RIGHT  NOW \G1\X0 WILL PASS THE CHARACTER.  THIS  WILL  BE

CHANGED LATER ON.

2. AUTO-RELIABLE BUFFER: \Cn AND \Jn COMMAND

In  auto-reliable mode (\N3), after the end of the  DSP  training

sequence  (CCITT  V.22  or  V.22  bis),  the  modem  attempts  to

establish an MNP "reliable" connection with the remote modem.  If

a  reliable connection is not possible (because the remote  modem

doesn't support it, or because the line is extremely noisy),  the

modem  falls back to a normal or a direct connection  (see  table

below). This process can take a few seconds.

The remote modem might start sending data before the local  modem

is  ready (for example if the remote modem doesn't support  MNP),

that  is, before the local modem has finished the  MNP  handshake

and decided to fall back to normal mode. Option \Cn decides  what

is going to happen to these characters received before the end of

the MNP handshake: (see flow charts)

If  \C0  is  selected,  the  characters  are  not  buffered.  Any

characters  sent by the remote modem before the local  modem  has

gone into data mode are lost. They are not sent to the local DTE.

If \C1 is selected, the data is buffered in the ANSWERING  modem.

If  the  buffer  fills  up  before  the  reliable  connection  is

established, the local modem goes into normal mode and passes the

contents  of  the buffer to the local DTE. The local  modem  also

goes  into  normal  mode if 4 seconds  elapse  without  making  a

successful MNP reliable connection.

If \C2 is selected, the data is not buffered. The modem  attempts

to  establish  a reliable connection for 4 seconds,  but  if  the

character defined by %A command (S37) is received from the remote

modem,  it switches immediately to normal mode. %A  character  is

recognized only by ANSWERING modems.

Note: the description below relates to the data received from the

remote modem over the phone line. The data from the local DTE  is

never  buffered  (  if a key is hit on the  local  terminal,  the

handshake is aborted.). %Cn setting is only valid in ANSWER mode.

In  ORIGINATE  mode,  the received data  is  never  buffered,  %A

characters  are  not recognized, and \Cn setting is  ignored:  it

makes sense because it is assumed that the answer modem won't try

to type before the originate modem.

When \N3 is selected, the final type of connection depends on the

remote modem and on options \C and \J. This is summarized in  the

following  table:  (the table describes the state  of  the  local

modem). The table is valid for ORIGINATE and ANSWER mode modems.

\N0, \N1	\N2, \N3   (remote modem)

-----------------------------------

|     normal       |     normal     |

\N0 |    ignore \C     |   ignore \C    |

|                  |                |

-----------------------------------

| direct           | direct         |

\N1 | ignore \C \J     | ignore \C \J   |

|                  |                |

-----------------------------------

|                  |                |

\N2 |  no connection   |      MNP       |

|                  |                |

-----------------------------------

|\C1,\C2: norm     |                |

\N3 |\C0: \J0 dir      |      MNP       |

|     \J1 norm     |                |

-----------------------------------

(local modem)

3. SPEED ISSUES: DTE SPEED ADJUST, CONNECT MESSAGE, HANG-UP

DTE speed adjust

SPEEDMODEM NOTE...DTE SPEED SHOULD BE FIXED AT 19,200 BPS,

NO AUTOBAUDING

When  establishing  a connection over the modem port,  the  final

type  of connection established depends on various factors,  such

as  the  type  of modem involved, the speed of each  of  the  DTE

connected to each modem, the condition of the phone line,  etc...

The final speed of the modem connection might not match the speed

of the DTE:

For example, suppose the local DTE is 2400 bps and the remote DTE

is 1200 bps. After establishing a normal or reliable  connection,

the  final speed of the connection will be 1200 bps (figures  [1]

and [2] ).

If  option \J0 is selected on the local modem (figure  [3]):  the

DTE port speed stays at 2400 bps. Flow control between the  modem

and the DTE will most likely be needed since the DTE will be able

to send data to the modem much faster than the modem will be able

to send them to the remote system.

If \J1 is selected (figure [4]), after the end of the  handshake,

the  modem will change the speed of the DTE port to 1200 bps.  If

the  DTE speed is not changed also to 1200 bps, the DTE  will  be

unable  to  communicate with the modem. The DTE will have  to  be

changed  to  1200. This will be done manually most of  the  time,

unless you are using a software that recognizes speed changes and

adjusts automatically.

Note

\Jn option is only valid for normal or reliable connection. In  a

direct  connection,  the  speed  of the  serial  port  is  always

adjusted,  regardless of the \Jn setting. It makes sense: if  not

buffering  and  flow control is possible, the speed  of  all  the

elements of the connection must be the same.

At hang-up, the old speed is never restored, except if \N1\J0  is

selected.

Using parallel modems:

If  \J1 is selected, the modem writes directly to the  UART,  and

there  is  no  need to give the "change  speed"  command  to  the

communication  software.  Note  that  in  that  case,  the  speed

displayed  by the communication software might not correspond  to

the actual speed of the DTE port.

Connect message

The message sent after a connection was established depends on  a

lot of factors as shown on the flow chart:

1)  If the connection is in direct mode, the messages  depend  on

the modem speed, which is also the DTE adjusted speed:

DTE adjusted BPS        direct connection

= modem speed

300             CONNECT

1200            CONNECT 1200

2400            CONNECT 2400

2) if X0 is selected, the message is always "CONNECT", no  matter

what.

3)  If  \V0  is selected, we use  the  standard  Hayes  messages,

depending on the modem speed:

modem speed     message

300             CONNECT

1200            CONNECT 1200

2400            CONNECT 2400

4) If \V1 is selected the message will reflect the DTE speed:

- If \J1 is active, the DTE speed is adjusted to match the  modem

speed (300, 1200 or 2400):

DTE adjusted bps        normal conn.    MNP connection

= modem speed

300             CONNECT CONNECT 0300/REL

1200            CONNECT 1200    CONNECT 1200/REL

2400            CONNECT 2400    CONNECT 2400/REL

-  If  \J0 is active, the connect message will  reflect  the  DTE

speed (the modem speed can be different)

DTE speed       normal  MNP

300             CONNECT CONNECT 0300/REL

1200            CONNECT 1200    CONNECT 1200/REL

2400            CONNECT 2400    CONNECT 2400/REL

4800            CONNECT 4800    CONNECT 4800/REL

9600            CONNECT 9600    CONNECT 9600/REL

Restoring UART speed at hang-up

For  normal and MNP connection, the previous UART speed is  never

restored:  if \J1 is selected and the UART speed was adjusted  at

the beginning of the connection, it stays the same after hang-up.

If \J0 is selected, the DTE speed is not adjusted, and  therefore

it never needs to be restored.

For direct connections: the DTE port speed is always adjusted, as

said  above.  But:  if  \J1 is active, the  DTE  speed  is  never

restored. If \J0 is active: the speed is restored if the  hang-up

occurs  when  not in command mode (you don't want to  change  the

port speed all of the sudden, if the user is in command mode).

For  synchronous  connections (&M1, &M2, &M3): the DTE  speed  ia

always restored.

4. BREAK COMMANDS

3 things to distinguish between:

1. Break received from local DTE

2. Break received from remote modem

3. send break command \B

1.a Break received from local DTE, during a direct connection

\K0,\K2,\K4     go into command mode

\K1,\K3,\K5     no action

In  a direct connection, break (scrambled space) will be  treated

as normal data, and therefore will be sent to the remote modem.

This is not implemented right now.

1.b Break received from local DTE, during a normal or reliable con.

\K0,2,4 Go to command mode (don't send break to remote)

\K1             Empty buffers and send break

\K3             Send break immediately

\K5             Send break in sequence with TX data

2.a Break received from remote modem during a normal connection

\K0,\K1 Empty buffers, send break to DTE

\K2,\K3 Send break immediately to DTE

\K4,\K5 Send break in sequence with RX data

2.b Break received from remote modem during direct  connections:

the action depends on Yn option (long space disconnect

3.a  Send  break  (AT\B) command, during  a  reliable  or  normal

connection

\K0,\K1 Empty buffers, send break to remote system

\K2,\K3 Send break immediately to remote system

\K4,\K5 Send break in sequence with TX buffered data.

3.a Send break (AT\B) command, during a direct connection

just send a break (there is no buffering problem here).

Comments

How do all the modes work ? In particular, what's the  difference

in 2.a between \K0,\K1 and \K4,\K5 ?

How do the other modes work ?

Need to add and document 2 functions for detecting break from DTE

and break from modem port5.

FACTORY SETTINGS AND EERAM

Three  factory  configurations are now  available:  AT&F  command

accepts  a  parameter: n=0, 1, 2. If n>2, ERROR is  sent.  If  no

parameter  is given, n=0 is assumed, this  ensures  compatibility

with the previous version. Factory configuration 0 is recalled at

power-up  if  no  EERAM  is  present,  or  using  command  AT&F0.

Configuration  1  and  2 are recalled by  using  commands  AT&F1,

AT&F2.

At  the first power-up (new EERAM), the EERAM configuration 0  is

loaded with factory configuration 0, and EERAM configuration 1 is

loaded with factory configuration 1.

&F0 - Hayes default. The modem behaves like a regular Hayes modem.

&F1  -  Microcom default. This is the default  configuration,  as

defined by Microcom.

&F2  - Auto-reliable configuration. This is a configuration  that

should suit most uses of MNP4/MNP5.

AT&F0 : "HAYES DEFAULT"

This  is the factory default as defined by Hayes: it is  recalled

at power-up, if no EERAM is present. S0 to S27 are as defined  by

hayes. The MNP registers are set to disable MNP operation.

Hayes registers

S0      0       disable auto-answer

S1      0       Ring count

S2      43      Escape code character '+'

S3      13      Carriage return

S4      10      Line feed

S5      8       Backspace

S6      2       Wait for dial tone

S7      30      Wait for carrier

S8      2       Pause for ',' dial

S9      6       Carrier detect response time

S10     14      Lost carrier to hang-up delay

S11     75      DTMF time (not Hayes)

S12     50      Escape code guard time

S14     0AAH    E1, Q0, V1, no dumb, Pulse dial, Originate

S16     80H     Enable auto-retrain

S18     0       Test timer

S21     20H     &J0, &R0, &D0, &C1, &S0, Y0

S22     76H     L2, M1, X4, &P0

S23     7       2400 bps, Even, &G0

S25     5       Delay to DTR

S26     1       RTS/CTS Delay

S27     40H     &M0, &L0, &X0, B1

MNP registers

S36     01H     \N1 (direct mode connection)

S37     00H     N/A

S38     00H     \J0 (restore DTE speed), \V0 (hayes results)

S39     00H     N/A

S40     00H     N/A

S41     00H     N/A

AT&F1 : "MICROCOM DEFAULT"

In  this  mode, the Hayes registers S0 to S27 are as  defined  in

&F0. The MNP registers are different:

MNP registers

S36     01H     \N1, \G0, \X0, \Q0

S37     00H     %A0

S38     24H     \C0, \J1, \E0, \V0

S39     00H     \T0

S40     A3H     \A3, \K5

S41     01H     %C1, &C0                if MNP5 EQU 1

00H     %C0, &C0                if MNP5 EQU 0

AT&F2 : MNP, AUTO-RELIABLE CONFIGURATION

The Hayes registers S0 to S27 are the same. The MNP registers are

as  follows:  auto  reliable mode is selected,  flow  control  is

enabled. The fall back character is set to a Carriage Return.

S36     47H     \N3, \G1, \X0, \Q1

S37     0DH     %A13

S38     32H     \C2, \J0, \E0, \V1

S39     00H     \T0

S40     A3H     \A3, \K5

S41     01H     %C1, &C0                if MNP5 EQU 1

00H	%C0, &C0		if MNP5 EQU 0

\C1     auto-reliable buffer

\G1     enable modem flow control

\J0     disable bps adjust

\N3     Auto-reliable mode

\Q1     XON/XOFF DTE flow control

\V1     enable MNP result codes

OPERATIONAL MODES - SUMMARY

(These  modes apply only if asynchronous mode is selected,  using

AT&M0.  If  &M1, &M2 or &M3 are selected, the modem  will  behave

like a regular Hayes synchronous modem)

\N0 - NORMAL MODE

If  \N0 is selected, the modem will only make normal  connections

(as opposed to \N3 setting, which can result in direct, normal or

reliable connections, depending on various factors).

For  normal  connections, flow control and  buffering  are  used.

There  is no MNP error correction. The relevant settings are  the

following (in order of importance):

\Gn  -  Modem port flow control : this setting is valid  only  in

normal mode.

\Qn - DTE port flow control

\Xn - XON/XOFF pass through

\Jn  - BPS rate adjust : \J0, the DTE port speed is  not  changed

when the connection is established, flow control might be needed.

\J1, the DTE port speed is adjusted to match the modem bps.

\En  - Data echo : this setting is only valid in normal mode.  E0

disables  the echo-ing of the data sent by the local DTE in  data

mode, E1 enables the echo.

\Kn - Break control

\Vn  - Connect message : if \V0 is used, the messages  are  Hayes

compatible  and reflect the modem speed (CONNECT,  CONNECT  1200,

CONNECT  2400).  If \V1 is used, the messages  will  reflect  the

(eventually  adjusted)  DTE speed: CONNECT  0300,  CONNECT  1200,

CONNECT 2400, CONNECT 4800, CONNECT 9600.

\Tn - Inactivity timer : (for now, only works in reliable modes)

%Dn - Disconnect buffer delay : (not implemented yet).

\N1 - DO NOT USE WITH SPEEDMODEM

COMMUNICATION SOFTWARE

(ABOUT FOREIGN LANGUAGES...)

Suppose the communication package you use is set to 8 data  bits,

no parity: if the remote DTE uses parity, the parity bit will  be

considered as the 8th data bit by your communication package, and

the  8  bit character sent to the terminal will appear  as  Greek

letters and other strange characters.

For ASCII only connections and file transfers:

If  the  local communication package is set to 8 data  bits,  the

remote communication package should also be set to 8 data bits (7

bits + space parity will also work). If the setting of the remote

modem  is unknown, or if it cannot be changed, you must set  your

local comm. package to 7 data bits.

General file transfers:

If  you want to be able to transfer binary files, both local  and

remote communication packages should be set to 8 data bits.

HOW TO TRANSFER FILES

USE OF COMMUNICATION PACKAGES:

NEW S REGISTERS

S36 REGISTER - FLOW CONTROL AND TYPE OF CONNECTION

bit 1,0  \Nn command

0 0     \N0     normal mode

0 1     \N1     direct (default)

1 0     \N2     reliable

1 1     \N3     auto-reliable

Bit 2   \G command

0        \G0     no modem flow control  for  normal  connections

(default)

1       \G1     use XON/XOFF flow control during normal connection

bit 3   \X command

0       \X0     don't pass through XON/XOFF (default)

1       \X1     pass through XON/XOFF

bit 5,4 Status of the connection

0 0     Normal  (no MNP buffered

0 1     Direct  (no MNP, non buffered)

1 0     MNP4 Reliable

1 1     MNP5 data compression

Note that these bits are meaningful only in asynchronous mode. If

a  synchronous connection is established, or if no connection  is

established, they must be ignored.

bit 7,6 DTE flow control

0 0     \Q0     no flow control (default)

0 1     \Q1     XON/XOFF bidirectional flow control

1 0     \Q2     CTS unidirectional flow control

1 1     \Q3     RTS/CTS bidirectional flow control

S37 REGISTER - AUTO-RELIABLE FALL BACK CHARACTER

bits 7 to 0

n       %An  n (between 0 and 127) will be recognized as the fall

back character in auto-reliable (\N3) mode. (default = 0). Bit  7

is always 0.

S38 REGISTER - MISCELLANEOUS OPTIONS

bits 1,0 \Cn command - Set Auto-reliable buffer

0 0     \C0 Do not buffer data (default)

0 1     \C1 Buffer data for 4 seconds

1  0     \C2  do not buffer data. Switch  to  normal  mode  when

character define by %An option is received.

1 1     not used

bit 2   \Jn - BPS adjust

0       \J0     disable bps adjust

1       \J1     Enable bps adjust (default)

bit 3   \En - Set data mode echo

0       \E0     don't echo in data mode (default)

1       \E1     echo in data mode, if non reliable

bit 4   \Vn - set MNP result codes

0       \V0     disable MNP result codes (default)

1       \V1     Enable MNP results

bit 5,6 free

bit 7

(reserved. Break request)

S39 REGISTER - INACTIVITY TIMER

S40 REGISTER - MISCELLANEOUS OPTIONS

bits 1,0 - \A command : maximum MNP block size for stream link con.

0 0     \A0     max block size = 64  characters  (default)

0 1     \A1     max block size = 128 characters

1 0     \A2     max block size = 192 characters

1 1     \A3     max block size = 256 characters

bits 2,4,3 free

bits 7,6,5 \K command - break control

0 0 0   \K0

0 0 1   \K1

0 1 0   \K2

0 1 1   \K3

1 0 0   \K4

1 0 1   \K5     (default)

S41 REGISTER - MISCELLANEOUS OPTIONS

bit 0   %Cn command

0       %C0     disable MNP5

1       %C1     enable MNP5 (default)

5.2 Most known result codes

-----------------------

A.1  RESULT CODES (See for more point 5.2)

Result  codes  are  the  responses  to  commands  that  have  been

executed. Result codes may be selected as English words or numeric

digits.   Word   result   codes  are  followed   by   a   carriage

return/linefeed  sequence; numeric result codes are followed by  a

carriage return only. Table 3-1 lists the result codes.

DIGIT     WORD            RESULT

CODE      CODE

0         OK              Command executed without error.

1         CONNECT         Carrier detected (300 bbs or 1200 bbs if

X0 is set.)

2         RING            Phone is ringing (will answer only if

auto answer is enabled.)

3         NO CARRIER      Carrier lost or never detected.

4         ERROR           Error in command line, line too long, or

invalid character format.

5         CONNECT 1200    Carrier detected at 1200 bbs if X1 set.

6         NO DIALTONE     Displayed if no dialtone is detected

within 5 seconds and X2 or X4 is set.

7         BUSY            Displayed when a busy signal is

detected and Z2 or X4 is set.

8         NO ANSWER       Displayed when "@" dial modifier is used

and phone continues to ring.

10        CONNECT 2400    Carrier detected at 2400 bbs. X1 or X2

must be set.

A.2  DATA TRANSMISSION RATE

The  Model  2400  will  transmit  and receive  data  at  the  rate

specified  by the table below. The transmission rate is determined

at the time of connection by three facrtors:  the data rate of the

data  terminal  connected to the Model 2400 modem; the  speed  and

standard of the remote modem; and whether or not the Model 2400 is

answering or originating the call.

ANSWERING MODEM

ORIGINATING

MODEL 2400     BELL 103    BELL 212    V.22        V.22 bis

300         1200        1200        2400

BELL 300       300         300         300**       300**

BELL 1200      1200*       1200        1200        1200

CCITT 1200     1200*       1200        1200        1200

CCITT 2400     2400*       1200        1200        2400

ORIGINATING MODEM

ANSWERING

MODEL 2400     BELL 103    BELL 212     V.22        V.22 bis

300         1200         1200        2400

BELL 300       300         1200         1200         2400

BELL 1200      300         1200         1200         2400

CCITT 1200     300*        1200         1200         2400

CCITT 2400     300*        1200         1200         2400

*  The connect rate shown results when a Model 2400 calls  another

Model  2400. The connect rate may differ when a Model 2400 is con-

necting with a different type of modem.

** Connection will be made if the answering modem is configured to

fallback  to  Bell 103. No connection will be made if fallback  is

set to V.21.

*OK                                     *1

*CONNECT                                *2

*RING                                   *3

*NO CARRIER                             *4

*ERROR                                  *5

*CONNECT 1200                           *6

*NO DIALTONE                            *7

*BUSY                                   *8

*NO ANSWER                              *48

*CONNECT 1200/75                        *49

*CONNECT 75/1200                        *12

*CONNECT 7200                           *14

*CONNECT 12000                          *15

*CONNECT 14400                          *20

*CONNECT 7200/MNP                       *22

*CONNECT 12000/MNP                      *23

*CONNECT 14400/MNP                      *28

*CONNECT 7200/MNP COMPRESSED            *30

*CONNECT 12000/MNP COMPRESSED           *31

*CONNECT 14400/MNP COMPRESSED           *36

*CONNECT 7200/V42                       *38

*CONNECT 12000/V42                      *39

*CONNECT 14400/V42                      *44

*CONNECT 7200/V42BIS                    *46

*CONNECT 12000/V42BIS                   *47

*CONNECT 14400/V42BIS                   *10

*CONNECT 2400                           *11

*CONNECT 4800                           *13

*CONNECT 9600                           *16

*CONNECT 300/MNP                        *17

*CONNECT 1200/MNP                       *18

*CONNECT 2400/MNP                       *19

*CONNECT 4800/MNP                       *21

*CONNECT 9600/MNP                       *24

*CONNECT 300/MNP COMPRESSED             *25

*CONNECT 1200/MNP COMPRESSED            *26

*CONNECT 2400/MNP COMPRESSED            *27

*CONNECT 4800/MNP COMPRESSED            *29

*CONNECT 9600/MNP COMPRESSED            *32

*CONNECT 300/V42                        *33

*CONNECT 1200/V42                       *34

*CONNECT 2400/V42                       *35

*CONNECT 4800/V42                       *37

*CONNECT 9600/V42                       *40

*CONNECT 300/V42BIS                     *41

*CONNECT 1200/V42BIS                    *42

*CONNECT 2400/V42BIS                    *43

*CONNECT 4800/V42BIS                    *45

*CONNECT 9600/V42BIS                    *60

*CONNECT 19200                          *61

*CONNECT 38400                          *62

*CONNECT 57600

******* Chapiter 6 - Manual MODEM6 / CALL-MODEM communication program **********

�������������������������������������������������������������Ŀ

�         Chapiter 6 : How to configure your modem            �

���������������������������������������������������������������

6.1  Modems avalaible on SETUP.EXE : Configurations

----------------------------------------------

Amiga 1200RS

Anchor Volksmodem 1200

Anchor 2400E

AT&T 2224 CEO - Auto Reliable

AT&T 2224 CEO - MNP-5 mode

AT&T 2224 CEO - MNP-4 mode

AT&T Model 4000

ATI 2400etc - Auto Reliable

ATI 2400etc - MNP-5 mode

ATI 2400etc - MNP-4 mode

ATI 2400etc V42 - Auto Reliable

ATI 2400etc V42 - V.42bis mode

ATI 2400etc V42 - V.42 mode

ATI 2400etc V42 - MNP-5 mode

ATI 2400etc V42 - MNP-4 mode

ATI 9600etc - Auto Reliable

ATI 9600etc - V.42bis Mode

ATI 9600etc - V.42 Mode

Avatech 2400E

BSM Quik Com MNP

Cardinal 2400 MNP

Cardinal 9650 V.32

Cermetek 1200 Modem/1200 SPC/Security Modem

Cermetek 2400 R/2400 SPC

Compaq Enhanced Internal Modem (V.42bis)

Companion CTS212AH

CompuCom Speedmodem

Concord Data CDS 224 Autodial

DataTrek 624 (UK)

DataTrek 624 (US)

DataTrek 2424 (MNP)

DataTrek V.32

Dell Data/Fax Modem

Delta Gold DM 1200

Digicom DSI9624

Digicom DSI9624 Plus

E-Tech Bullet PC2400MH

E-Tech Bullet E2400M

E-Tech Bullet E9696M

E-Tech Bullet PC9696M

Everex Evercom 12e

Everex Evercom 24e

Everex II-24

Everex EV941

Everex Evercom 24e+ (MNP 5)

Everex Carrier 24

Everex Carrier 24/96

Everex EverFax 24/96E

Everex 96E+

Fastcomm 2496/Turbo

Fastcomm 9696

Forval IM14400

General DataComm Desktop 596

GVC Super Modem 2400 MNP-

GVC Super Modem 9600 V.32

Hayes-Compatible 1200 Baud Modem

Hayes-Compatible 2400 Baud Modem

Hayes Personal Modem 1200

Hayes Personal Modem 2400

Hayes Smartmodem 1200/1200B

Hayes Smartmodem 2400/2400B

Hayes Smartmodem V Series 2400/2400B

Hayes Smartmodem V Series 9600/9600B/9600 V.32

Hayes Optima 9600

Hayes Ultra 9600

Hayes Ultra 14400

Hayes V Series 2400/2400B V.42

Hayes V Series 9600/9600B V.42

Hayes ISDN Terminal Adaptor

IBM Modem 1200 (5841)

IBM PC 2400 bps Modem

IBM PS/2 Data/Fax Modem

IBM 7855 Model 10

Image Twincom 96/24i Auto-Reliable mode

Image Twincom 96/24i V.42bis mode

Image Twincom 96/24i V.42 mode

Image Twincom 96/24i MNP-5 mode

Image Twincom 96/24i MNP-4 mode

InfoMate 212A/PC

InfoMate 212X/PC

Intel 2400B

Intel 2400B MNP

Intel 2400EX MNP

Intel 9600EX

Intel 14400EX Auto-Reliable mode

Intel 14400EX V.42bis mode

Intel 14400EX V.42 mode

Intel 14400EX-MNP-5 mode

Intel 14400EX MNP-4 mode

Intel SatisFAXtion Board

Maxwell Modem 300PC/300V

Maxwell Modem 1200PC/1200V

The Maxwell Modem 1200VP

The Maxwell Modem 2400PC

MegaHertz EasyTalk 1200

MegaHertz EasyTalk 2400

Microcom AX/2400 MNP4

Microcom AX/2400c MNP5

Microcom AX/9612c

Microcom AX/9624c

Microcom QX V.32c

Microcom QX 2400t

Microcom QX/4232hs

Miracom Keycard 2400

Miracom Keycard 3000

Miracom WS4000/PC

Motorola-Codex 2264

Codex 3220 Auto-Reliable mode

Codex 3220 MNP-5 mode

Codex 3220 MNP-4 mode

Motorola-Codex 3260 Series

MultiTech MultiModem 224/224PC

MultiTech MultiModem 224E/224EC

MultiTech MultiModem 224E7 V.42bis

MultiTech MultiModem V.32

MultiTech MultiModem V.32 EAB V.42bis

MultiTech MultiModem 696E

NEC N2431/2431C

Novation Professional 2400

Novation Smart-Cat 103/212

Okidata CLP 296

Okidata Okitel 2400 Plus/2400B Plus

Okidata Okitel 1200/1200B

Okidata Okitel 2400/2400B

Okidata Okitel 9600

Pace Linnet 1200

Pace Linnet PC Card (300 baud)

Pace Series Four

Pace Ultralink Quad

Pace Ultralink 32

Penril Datalink 1200

Penril Datalink 2400

Penril Alliance V.32 Series

Practical Peripherals Practical 1200

Practical Peripherals Practical 2400

Practical Peripherals 2400SA

Practical Peripherals 2400 Pocket Modem

Practical Peripherals 2400SA MNP

Practical Peripherals 2400SA V.42bis

Practical Peripherals PM9600SA V.32

Premier Innovations P2400 / P2400E

Prometheus Pro Modem 1200

Prometheus 2400 MCT-24I Half-card

Prometheus 9600 MNP

Prometheus ProModem Ultima

Racal-Milgo RMD 3221

Racal-Vadic 1200PA

Racal-Vadic 2400/PS

Racal-Vadic 2400LC

Racal-Vadic LC2400PC

Racal-Vadic 2400PA

Racal-Vadic 2400PA Model 2

Racal-Vadic 2400VP

Racal-Vadic 9600VP

Racal-Vadic 9632PA

Racal-Vadic VA212

Racal-Vadic VA3451

Supra 2400 MNP

Supra 2400 Plus (V.42bis)

Supra 9600

Sysdyne MDM 24H

Tandy 1200-bps Half Card

Telebit Internal PC Card w/ MNP

Telebit T1000

Telebit T1500 - V.42 mode

Telebit T2500 - V.42 mode

Telebit T1500 - MNP mode

Telebit T2500 - MNP mode

Telebit T1600

Telebit T2000

Telebit T3000 (57.6K baud)

Telebit T3000 (38.4K baud)

Telebit Trailblazer / Trailblazer Plus

Telebit Q-Blazer Auto-Reliable mode

Q-Blazer V.42bis mode

Q-Blazer V.42 mode

Q-Blazer MNP-5 mode

Q-Blazer MNP-4 mode

UDS Fastalk 2400

UDS Fastalk V.32/V.42

UDS V.3224/V.3225

UDS V.3227

US Robotics Sportster 1200/1200 PC

US Robotics Sportster 2400/2400 PC

US Robotics 2400PC

US Robotics 2400PC MNP

US Robotics Courier 2400

US Robotics Courier 2400e/ 2400e/ps

US Robotics Courier 2400PC

US Robotics Courier HST

US Robotics Courier HST V.42

US Robotics Courier HST-ASL

US Robotics V.32

US Robotics V.32bis-ASL

US Robotics Dual Standard

USR Dual Standard V.32bis - Auto Reliable mode

USR Dual Standard V.32bis - V.42bis mode

USR Dual Standard V.32bis - V.42 mode

USR Dual Standard V.32bis - MNP-5 mode

USR Dual Standard V.32bis - MNP-4 mode

Ven-Tel 212Plus

Ven-Tel Halfcard

Ven-Tel Halfcard 24

Ven-Tel 2400 Plus II

Ven-Tel 9600 Plus / Plus II

Ven-Tel PCM2400E

Ven-Tel Pathfinder

ViVa 24 & 2496if

ViVa 24fx & 24i/fx

ViVa 24m

ViVa 24im

ViVa 2442i

ViVa 9642e

Zeos Notebook Modem

Zoom Modem PC 1200

Zoom Modem PC 2400

Zoom 2400 V.42bis

Zoom V.32 Turbo

Standard   300

Standard  1200

Standard  2400

Standard  4800 MNP/V42

Standard  9600 MNP/V42

Standard 14400 MNP/V42

******* Chapiter 7 - Manual MODEM6 / CALL-MODEM communication program **********

�������������������������������������������������������������Ŀ

�              Chapiter 7 : HAYES codes index                 �

���������������������������������������������������������������

7.1  HAYES codes Index

-----------------

A Manually answer incoming call

A/ Repeat last command executed

AT Appear at the beginning of every command

B B0= Variable link rate CCITT compatibility

B1= variable link rate BELL compatibility

B2= Autoscan V.32/V.22BIS/V.22

B3= 1200/75 bps CCITT V.23 only

B4= 300 bps

B5= 1200 bps

B6= 2400 bps

B7= 4800 bps

B8= 9600 bps

B9= 14400 bps

D Dial command

DS Dials one of the 4 Tel. numbers stored in memory

E E0= commands are not echoed

E1= commands are echoed

+++ Switch data mode to command mode

H H0= Modem is on hook

H1= Modem is off hook

I I0= Display product code

I1= Display checksum code

I2= Internal memory test

I3= Display MNP ID code

I4= Display Version code

L L1= Lower speaker volume

L2= Medium speaker volume

L3= Loud speaker volume

M M0= Speaker off

M1= Speaker on until on-line

M2= Speaker always on

M3= Speaker on until online and off while dialing

O O0= Return to online

O1= Initiate an equalizer sequence

P Pulse dialing

Q Q0= Send response

Q1= Do not send response

Sr? Read value in register r

Sr=N Set register r to value N

T Tone dialing

V V0= Digit responses

V1= Word responses

X X0= Hayes smartmodem 300 compatibility

X1= Include connect speed responses

X2= Include dialtone detection responses

X3= Include busytone detection responses

X4= Include both dialtone and busytone detection response

Y Y0= Long space disable

Y1= Long space enable

Z Z0= Reset and retrieve active profile 0

Z1= Reset and retrieve active profile 1

& commands :

------------

&C &C0= DCD Signal always on

&C1= DCD signal follows modem CD signal

&D &D0= Ignore DTR signal

&D1= Return to command mode when DTR on-to-off transition

&D2= Hang up when DTR on-to-off transition

&D3= Reset when DTR on-to-off transition

&F &F0= Recall normal configuration

&F1= Recall MNP 5/V.42bis configuration with XON/XOFF

&F2= Recall MNP 5/V.42bis configuration with RTS/CTS

&G &G0= Guard tone disabled

&G1= 550Hz Guard tone

&G2=         1800Hz Guard tone

&H &H0= Display help manual of standard commands

&H1= Display help manual of MNP 5/V.42bis commands

&L &L0= Dialup operation

&L1= Leased-line operation

&M &M0= Asynchronous only

&M1= Modem enter synchronous mode after dialing

&M2= Synchronous only

&M3= Manually originating a synchronous call

&O &O0= Perform like a normal modem

&O1= Perform like an originate-only modem

&P &P0= M/B ratio=39/61

&P1= M/B ratio=33/67

&R &R0= CTS follows RTS

&R1= CTS always on

&S &S0= DSR always on

&S1= DSR follows RS-232C specification

&T &T0= Ends test in progress

&T1= Local analog loopback test

&T3= Local digital loopback test

&T4= Remote digital loopback test enabled

&T5= Remote digital loopback test disabled

&T6= Remote digital loopback test

&T7= Remote digital loopback with self test

&T8= Local analog loopback  with self test

&V Display modem profiles

&W &W0= Stores profile 0

&W1= Stores profile 1

&X &X0= Synchronous clock provided by modem

&X1= Synchronous clock provided by EIA

&Y &Y0= Profile 0 will be retrieved when power on

&Y1= Profile 1 will be retrieved when power on

&Zn Stores up to 4 telephone numbers in memory

% commands :

------------

%D %D0= Send Clear-Down disabled

%D1= Send Clear-Down enabled

%Ln Set transmit level -n dBm

%An Set auto-reliable fallback character to n

%C %C0= Disable MNP 5 or V.42BIS data compress

%C1= Enable MNP 5 or V.42BIS data compress

\ commands :

------------

\A \A0= 64 Character MAX. MNP block size

\A1= 128 Character MAX. MNP block size

\A2= 192 Character MAX. MNP block size

\A3= 256 Character MAX. MNP block size

\Bn Send n*1/10 second line break to modem

\C \C0= Do not buffer data

\C1= Buffer all data for 4 seconds

\C2= Switch to normal mode when fall back character is detected

\E \E0= Do not echo data during a normal link

\E1= Echo data during a normal link

\G \G0= Disable DCE flow control

\G1= Enable DCE flow control

\H \H0= HP ENQ/ACK protocol disable

\H1= HP ENQ/ACK protocol enabled, modem to be as host

\H2= HP ENQ/ACK protocol enabled, modem to be as peripheral

\J \J0= Turn off DTE baud rate adjustment

\J1= Turn on DTE baud rate adjustment

\Kn \K0,1= Expedited destructive

\K2,3= Expedited nondestructive

\K4,5= Queued

\N \N0= Normal data link only

\N1= Direct data link only

\N2= Reliable MNP data link only

\N3= Auto-reliable MNP link

\N4= Reliable V.42 data link only

\N5= Reliable V.42 or MNP data link only

\N6= Auto-reliable V.42 or MNP data link

\O Initiate reliable link during a normal link

\Q \Q0= Turn off flow control

\Q1= XON/XOFF flow control

\Q2= CTS undirectional flow control

\Q3= RTS/CTS flow control

\Q4= Uni-directional XON/XOFF

\Tn Inactivity timer

\U Accept reliable during a normal link

\V \V0= Do not send extended responses

\V1= Send extended responses

\X \X0= Process XON/XOFF and do not pass through

\X1= Process XON/XOFF and pass through

\Y Switch to reliable mode from normal link

\Z End the reliable link and switch to normal operation

******* Chapiter 8 - Manual MODEM6 / CALL-MODEM communication program **********

�����������������������������������������������������������������Ŀ

� Chapiter 8 : Tutor : How to connect to another system via modem �

�������������������������������������������������������������������

8.0  First steps

-----------

HOST INTERFACE

This section describes the way in which the host interfaces to the

modem.  The  modem communicates to the host in either the  command

mode  or the data mode. The Model 2400 controls the switching  be-

tween  the  two modes. The staus of the modem is communicated  via

result codes.

COMMUNICATION MODES

The  modem communicates to the host in one of two possible  modes:

the  command  mode or the data mode. Upon powerup or after  reset,

the  modem  performs a selftest. Immediately following this  test,

the  modem  enters the command mode and configures itself  in  the

default  configuration. The modem then enters the autobaud routine

and waits for the attention code 'AT' from the host.

A.1.1  Command Mode

In  the command mode, the user communicates with the modem via the

host  computer.  This  mode  is used for  modem  configuration  or

dialing.  The modem executes local commands from the host computer

and  returns  the  result codes. After the autodialing  and  modem

handshake sequences are completed, the modem enters the data mode,

unless otherwise specified by the dial subcommand.

A.1.2  Data Mode

In  the data mode, the modem receives characters from the host and

processes  the data before transmitting it to the telephone  line.

The  modem  also checks the data from the host for an escape  code

sequence. It does not execute any local commands in the data mode.

The modem returns to the command mode only when it receives an es-

cape code sequence from the host, or one of the data call termina-

tion procedures described in Section  6.5 occurs.

A.2  MODE CONTROL

Upon  powerup,  the Model 2400 modem is in the command  mode.  The

modem  can enter the data mode while connected to another modem or

when  using diagnostics. The escape code is used to return to  the

command  mode.  To return back to the data mode, the ATO  (online)

command is used.

The  escape  code forces the modem to return to the  command  mode

from  the data (online) mode. The escape code character and escape

code guard time are defined by the values set in register S12. The

escape  code  guard time is defined as the time delay between  the

last  character  of  escape  code transcitted and  the  next  data

character. The default guard time is defined as the time delay be-

tween  the last character of escape code transmitted and the  next

data  character.  The  default guard time is one  second  and  the

default  code character is "+". The escape code character must  be

entered  three  consecutive  times.  To execute  the  escape  code

(default values), proceed as followe:

1)  Type "+++"

2)  Wait one second before transmitting another command character.

The  modem  returns to the command mode and sends the result  code

"OK".  The  telephone  line connection is retained or  aborted  as

defined  by  the value in register S10. The connection is  aborted

automatically if the carrier signal from the remote modem is lost.

If  the  modem is forced into the command mode from the data  mode

using the escape sequence, the ATO (online) command can be used to

return  to  the  data mode. The modem can be reconfigured  to  the

default state by executing the software reset ATZ command.

8.1  Communication - Introduction

----------------------------

First step to communicate with another system is to prepare connections,

after that you load MODEM6 program and use Option "Alt C". (Connection)

8.2  Connections

-----------

* Internal Modem :

����������Ŀ

Line  In     ->� Internal �          <--     Connect "Line In" to mural

Phone / Out  ->� Modem    �              PTT connector, "Phone / Out" to

��______����              optional phone.

�     �     �

�     �     �

\_/   \_/   \_/

��������������������Ŀ      <--     Plug Internal modem card to

�                    �           a 16 or 8 bits bus on your

�   PC Compatible    �           computer.

�                    �

����������������������

�

��������������������Ŀ      <--     Write "MODEM6" on DOS prompt

�    PC Keyboard     �           to load communication program

����������������������

* External Modem :

Do same as mentionned above, and plug a Rs-232 cable to a COM port on

back of your computer (like for your mouse) and on back of the modem.

8.3  Software control

----------------

8.3.1 Load and dial number from Phone Book

------------------------------------

Load MODEM6 program with correct configuration. (Baud, Parity, Data,

and COM port).

When you have the Main menu on screen press "Alt C", to call Phone Book

option. After that, choose your BBS number with "#/#" and validate with

"#�" key.

Example of the screen :

-----------------------

������������������������������������������������������������������������������ͻ

�   N�  Name                      NUMBER          BAUD   P D A D     ORDER     �

������������������������������������������������������������������������������Ķ

� # 1  Transnet (Gen�ve CH)     733 80 50          2400  N 8 1 F  ___________  �

�   2  Gonisoft (Node 1)        757 65 73          2400  N 8 1 F  ___________  �

�   3  Gonisoft (Node 2)        757 65 87          9600  N 8 1 F  ___________  �

�   4  Sun Server (Gen�ve)      300 35 80          9600  N 8 1 F  ___________  �

�   5  Sun Server (Gen�ve)      300 45 48          9600  N 8 1 F  ___________  �

�   6  CCTI (Gen�ve 9600)       781 51 89          9600  N 8 1 F  ___________  �

�   7  CCTI (Gen�ve 2400)       781 51 41          2400  N 8 1 F  ___________  �

�   8  SwissAir                 049 047 111C       9600  N 8 1 F  ___________  �

�   9  Central Point Software   001 503 6906650    2400  N 8 1 F  ___________  �

�   10 Wayne Bell (California)  001 213 2086689    9600  N 8 1 F  ___________  �

�   11 Tyffany (California)     001 510 4586404    9600  N 8 1 F  ___________  �

�   12 Omen (Usa)               001 503 6213746    9600  N 8 1 F  ___________  �

�   13 Deep Thought (Usa)       001 512 2441598    9600  N 8 1 F  ___________  �

�   14 Flushot + (Usa)          001 914 5862025    9600  N 8 1 F  ___________  �

������������������������������������������������������������������������������Ķ

� PgUp  Scroll up     Home First page     P  Print index       C  Change data  �

� PgDn  Scroll dn     End  Last page     *M  Manual number    Spc Change data  �

� Esc   End, exit     #/#  To choose      #� Dial number      *D  Delete data  �

������������������������������������������������������������������������������Ķ

� Message :                            � 38400 COM2 N 8 1 � Port Address : 760 �

������������������������������������������������������������������������������ͼ

8.3.2 Results of Dialing order

------------------------

Will appear on screen the messages below, and you will hear modem

dialing phone number that you choosed.

See example below for more informations,

Example of the screen :

-----------------------

������������������������������������������������������������������������������Ŀ

�     ATTENTION : I'm going to be connected !...    Wait a moment, Please !    �

��������������������������������������������������������������������������������

AT Z

OK

AT M1

OK

AT D  781 51 89

8.3.3 Connection to choosed BBS : Phone number => 781 51 89

-----------------------------------------------------

Like we choosed number 781 51 89, we are connecting with a Swiss BBS

called CCTI, in Geneva city (Switzerland).

After a CONNECT message you will see screen below, that is their

own CCTI presentation, when we access to the BBS.

Example of the screen :

-----------------------

<------------------                     3 Nodes: 781.51.41  ->14400Bps V42b

<-----------------/ BIENVENUE.......              781.51.89  ->14400Bps V42b

<----------------//------------------/             320.56.88  ->2400Bps

//------------------//     ===================================

/------------------///      Utilisateurs a 2400 Bps merci

**********************          ///       d'utiliser en priorite la ligne

* Sur le serveur du  *         ///        a 2400 Bps (320.56.88) Merci !

*  COMPUTER CLUB TI  *        ///         ===================================

*      Siege :       *       ///

* 5, rue du Vidollet *      ///-->        Vrais noms uniquement SVP, les

*    1202 GENEVE     *     //--->         donnees sont controlees !

* Tel : 022 7348515  *    /---->

* (infos club uniqu.)*

**********************

*Sysop : Eric Schefer*

**********************

ATTENTION : limitations de telechargement, voir rubrique bulletins du CCTI !!

=============================================================================

To access to this BBS, we enter our name, birth name, and password.

If all is valid, we can use services of CCTI BBS.

Below, you will see the Main Menu of the BBS, that is in french

language.

Example of the screen :

-----------------------

��������������������������������������������������������������������������������

���������������������������������������������������������������������������Ŀ���

����<CCTI>   19,200 Bps ������ Menu Principal ������    Node: 2     <Gen�ve>� ��

����������������������������������������������������������������������������� ��

����                                                                          ��

��������������������������������������������������������������������������������

������                                                                    ������

������    ��[G�n�ral]�����������������������������������������������Ŀ     �����

������    � M - Messagerie                    I - Transferts IBM PC  �     �����

������    � T - Transferts TI99/4A            A - Transferts Amiga   �     �����

������    ��[Communications]����������������������������������������Ĵ     �����

������    � Q - Qui a appel�                  W - Qui est en ligne ? �     �����

������    � D - Communication en Direct                              �     �����

������    ��[Divers]������������������������������������������������Ĵ     �����

������    � * - Infos membres                 B - Bulletins          �     �����

������    � E - Temps �coul�                  U - Utilitaires        �     �����

������    � S - Sondages                      C - Appeler le Sysop   �     �����

������    � G - Goodbye                       R - Rubrique adultes   �     �����

������    ������������������������������������������������������������     �����

������                                                                     �����

��������                                                                   �����

��������������������������������������������������������������������������������

Using option "I - Transferts IBM PC", key I, it's possible to access

to file Download/Upload service. See screen below.

Example of the screen :

-----------------------

��������������������������������������������������������������������������������

���������������������������������������������������������������������������Ŀ���

����<CCTI>   19,200 Bps  ������ Menu IBM PC ������      Node: 2     <Gen�ve>� ��

����������������������������������������������������������������������������� ��

����                                                                          ��

���                                           ����������������������������������

���    ��[Transferts]�������������������Ŀ     ���������������������������������

���    � P - Prendre des Programmes      �     ���������������������������������

���    � D - Donner des Programmes       �     ���������������������������������

���    � N - Nouveaux Fichiers           �     ���������������������������������

���    � A - Listes de Fichiers Publics  �     �������߰�����߰��߰���߰��������

���    � R - Sharewares am�ricains       �     ������߰���߰��߰��߰��߰��������

���    ��[Messages]���������������������Ĵ     ������߰���߰��߰������߰��������

���    � S - Scan des messages PC        �     ������߰������߰��߰߰�߰��������

���    � L - Lire les messages           �     ������߰���߰��߰�߰���߰��������

���    � E - Ecrire un message           �     ������߰���߰��߰�߰���߰��������

���    ��[Divers]�����������������������Ĵ     �������߰�����߰��߰���߰��������

���    � V - Voir son Ratio Up/Download  �     ���������������������������������

���    � -   Retour au Menu Principal    �     ���������������������������������

���    � G - Goodbye  T - Temps Ecoul�   �     ���������������������������������

���    �����������������������������������     ���������������������������������

���                                            ���������������������������������

***    You connected to CCTI BBS, using MODEM6 program.   ***

******* Chapiter 9 - Manual MODEM6 / CALL-MODEM communication program **********

�������������������������������������������������������������Ŀ

�    Chapiter 9 : Using SETUP program to configure MODEM6     �

���������������������������������������������������������������

9.1  First menu : Choosing modem type-configuration

----------------------------------------------

With menu below, you can choose your modem on 225 different types

already installed on the system. Like that configuration line will be

optimum for your modem.

MODEM6 / CALL-MODEM SETUP

1     Amiga 1200RS

2     Anchor Volksmodem 1200                             ������������������Ŀ

3     Anchor 2400E                                       �   M O D E M 6    �

4     AT&T 2224 CEO - Auto Reliable                      �    s e t u p     �

5     AT&T 2224 CEO - MNP-5 mode                         �  p r o g r a m   �

6     AT&T 2224 CEO - MNP-4 mode                         ������������������ĳ

7     AT&T Model 4000                                    �   Choose modem   �

8     ATI 2400etc - Auto Reliable                        � type and press S �

9     ATI 2400etc - MNP-5 mode                           � to enter modem # �

10    ATI 2400etc - MNP-4 mode                           � and save data on �

11    ATI 2400etc V42 - Auto Reliable                    � disk.            �

12    ATI 2400etc V42 - V.42bis mode                     ������������������ĳ

13    ATI 2400etc V42 - V.42 mode                        �                  �

14    ATI 2400etc V42 - MNP-5 mode                       � MODEM NUMBER :   �

15    ATI 2400etc V42 - MNP-4 mode                       �    �������Ŀ     �

16    ATI 9600etc - Auto Reliable                        �    �  ���  �     �

17    ATI 9600etc - V.42bis Mode                         �    ���������     �

18    ATI 9600etc - V.42 Mode                            ��Function Keys���ĳ

19    Avatech 2400E                                      � PgUp  Scroll up  �

20    BSM Quik Com MNP                                   � PgDn  Scroll down�

21    Cardinal 2400 MNP                                  � Q     quit setup �

22    Cardinal 9650 V.32                                 ��������������������

23    Cermetek 1200 Modem/1200 SPC/Security Modem

(C) JanSoft International - Geneva - Switzerland

9.2  Second menu : Configuring COM port, Baud rate, Parity, ...

----------------------------------------------------------

On this second menu, you can configure "Com Port", modem "Speed",

"Data bits", "Stop Bits" and "Parity".

Pressing on "C", "E", "D", "S" or "P" you can choose the right

parameter to configure.

When all configuration is done, press "RETURN" to quit SETUP program

and to save configuration on "MODEM6.CFG" file.

MODEM6 / CALL-MODEM SETUP

����������������������������������������������������������������������������Ŀ

�                   Telecommuncation program under Ms-Dos                    �

�                Quickly also under Ms-Windows from Microsoft !              �

�                     Now, made easy telecommunication !                     �

�        �������������������������������������������������������������Ŀ     �

�        �                �  �Ŀ ڿ� �Ŀ �Ŀ �� ���                    �     �

�        �                �  �Ĵ �� �Ŀ � � ��  �                     �     �

�        �              ���  � � � � ��� ��� �   �   Vers.             �     �

�        ���������������������������������������������������������������     �

����������������������������������������������������������������������������Ŀ

� Modem configuration :  (Press RED letter to configure data with keys #/#)  �

� ---------------------                                                      �

�  Com port    o COM1     spEed   o   110             Data bits  o  5        �

�              * COM2             o   300                        o  6        �

�              o COM3             o  1200                        o  7        �

�              o COM4             o  2400                        *  8        �

�                                 o  4800       Parity  *  N                 �

�  Stop bits   * 1                o  9600               o  E                 �

�              o 2                o 19200               o  O                 �

�                                 * 38400               o  M                 �

�                                 o 57600               o  S                 �

�� Press RETURN to save and quit ���������������������������������������������

(C) JanSoft International - Geneva - Switzerland

******* Chapiter 10 - Manual MODEM6 / CALL-MODEM communication program *********

��������������������������������������������������������������Ŀ

� Chapiter 10 : More informations about us and MODEM6 software �

����������������������������������������������������������������

10.1  What's new about ?

------------------

* This software had been developped using more than 3 years of free time

and is more than 6.5 Mb of source code and libraries to emulate all

best known features available.

It Supports :

- protocols like ZMODEM, YMODEM, XMODEM, KERMIT...

- Ansi, Ascii terminal emulation (Next VT series !)

- Speeds from 110 bauds to 57600 (Next 115200 bps)

- Parity (N/E/O/M/S)

- Data bits (5/6/7/8)

- Stop bits (1/2)

- Communication ports (COM1/COM2/COM3/COM4)

This software is not error free, but almost all software parts had been

tested before international distribution.

* About Software upgrades :

�� SHAREWARE NOTIFY ����������������������������������������������������������Ŀ

�                   Telecommuncation program under Ms-Dos                      �

�               Quickly also under Ms-Windows from Microsoft !                 �

�                   "Now, made easy telecommunication !"                       �

�        �������������������������������������������������������������Ŀ       �

�        �                �  �Ŀ ڿ� �Ŀ �Ŀ �� ���                    �       �

�        �                �  �Ĵ �� �Ŀ � � ��  �                     �       �

�        �              ���  � � � � ��� ��� �   �   Vers.             �       �

�        ���������������������������������������������������������������       �

�  If you find MODEM6 fast, easy, and convenient to use, a registration of $25 �

�  would be appreciated.  If you send $50 or more you will receive, when       �

�  available, each 6 months new version of the MODEM6 program. (3 times)       �

�                                                                              �

�  Please state the version of the software that you currently have.  Send     �

�  postal check, cash or international money order to :                        �

�                                                                              �

�                          Jan SAGGIORI                                        �

�                          "Les m�sanges"                                      �

�                          5, chemin de la chevillarde                         �

�                          1208 GENEVE (Switzerland)                           �

�                                                                              �

�  T�l : (41-22) 348 77 55     Fax : (41-22) 349 86 74   Fidonet : 2:301/327   �

��������������������������������������������������������������������������������