ACTSTIME

ACTSTIME is a program that utilizes the Automated Computer Time

Service (ACTS) provided by the National Institute of Standards

and Technology (NIST). ACTS is a dial-up service which provides

extremely accurate time and date information. The source for the

time information is NIST's Atomic Cesium clock located in Bolder

Colorado. This same clock is used as the reference for NIST's

shortwave radio stations WWV in Ft Collins, CO. and WWVH in

Kauai, Hawaii. NIST also provides other time, frequency, and

satellite services, all referenced to this Atomic clock.

ACTSTIME operates with a standard "Hayes compatible" AT command

set modem. The ACTS service operates at 300 and 1200 baud, so

virtually any modem should work with this program.

When this program is executed, a call is placed to ACTS in

Colorado. Naturally this is a long distance call to most users of

this program, but the duration of the call is quite short. Once a

successful connect is made, this program will be connected for

less than 5 seconds. For most callers, this would cause a billing

of 1 minute. Done at night, the cost of this call is

approximately 10 cents for most LD services, which is about the

same as calling your "local" 976-TIME number (which is an audio

service, not an computer based service). Done four times a month

(say, weekly), this program will keep your computer's clock

accurate for about 40 cents. Depending on how much your clock

drifts in a week, you should always be within several seconds of

"real" time.

For batch file, and BBS operation (in event files), ACTSTIME can

be instructed to only place the call on certain days of the week.

This method of operation will be explained later.

The Configuration file

The file ACTSTIME.CFG is expected to reside in the SAME directory

that ACTSTIME is executed from. The configuration file contains

eight (8) lines, each terminated by a carriage return. No

comments are allowed on the lines, only the configuration data.

A typical configuration file looks like this (and does not

necessarily match the one that came with this program):

1

0

38400

E0

ATDT1303-494-4774

05:00:00

1

5

Line 1 is the serial port that ACTSTIME is to use. In the case of

the example above, this is COM1. This can be any number from 1 to

4, or a decimal value representing the base address of a non-

standard comm port (example: non-standard port address 768 hex

would be 1896 decimal, which is what you'd enter on this line).

Line 2 is the IRQ for the port. If line 1 is 1 - 4, then line 2

should be 0 unless a non-standard IRQ is being used. If line 1 is

a decimal base address (other than 1 - 4), then line 2 must

include the IRQ value for the port.

Line 3 is the baud rate to be used with the modem. This can be

any rate that your modem supports. Remember however that the ACTS

service is 300 or 1200 baud. In the case of the above example,

38400 is being used as the port speed. The modem will still

connect at 1200. Make sure your modem DOES NOT do an "autobaud"

to the connect speed. It must remain locked at the rate you open

the port at. If your modem's serial speed cannot be locked at a

given rate (as is the case with many 2400 baud modems) then you

should specify 1200 for the baud rate on line 3.

Line 4 is the modem initialization command. "E0" is the AT

command for "Echo OFF". This program must operate with the

modem's Echo turned off, or it won't function properly. You can

add other commands to this line if you like, but E0 should remain

in the init string. The modem must also operate in VERBOSE mode

so that call progress strings such as "BUSY", "CONNECT" (etc.)

appear. This is usually the V1 command for "verbose" mode, and

the "Xn" command for the call progress strings (where n is a

number). Check your modem documentation if your commands are

different.

Line 5 is the dial command. Leave this line alone unless you need

to add other commands such as dialing "9" first, adding LD

service codes, country codes, or dialing in pulse mode if you

don't have Touch-Tone service (ATDP). There is only 1 number for

ACTS, and this is it. In case you loose the configuration file,

the telephone number for ACTS is 303-494-4774.

Line 6. The time given from ACTS is "Universal Coordinated Time"

or UTC for short. Years ago this was referred to a Greenwich Mean

time, and is still called Zulu time (as in 03:45Z). Basically it

is the time at the Zero Meridian (Longitude 0 degrees), which

also happens to pass through Greenwich, England. For any location

on Earth, UTC is the same time (UTC has no time "zones").

However, for local time, a positive or negative offset must be

applied. Line 6 is the offset value, specified in hours, minutes,

and seconds. Since most offsets are in whole hours (no minutes

and seconds), an offset of 5 hours would be "05:00:00" as shown

above. This offset value is treated as a NEGATIVE offset from

UTC. Since this program was only designed to operate in the US,

Canada, Alaska, and Hawaii, this is not a problem since the

offset to any of these places is always negative. The offset is

always negative for all WEST longitudes, and positive for all

EAST Longitudes. Typical offsets are 04:00:00 for Atlantic time

(New Foundland and New Brunswick), 05:00:00 for Eastern Time,

06:00:00 for Central Time, 07:00:00 for Mountain Time, and

08:00:00 for the Pacific time. Alaska and Hawaii are 10:00:00. DO

NOT include an adjustment for Daylight Savings time (which

reduces these values by 1 hour). That is calculated automatically

by this program (See line 7).

Line 7 is "Use Daylight Savings Adjust". Set to a 1, the proper

adjustment is made for Daylight Savings when it is in effect.

This information is provided by ACTS. When set to 0, the offset

will always be as shown on line 6.

Line 8 is an "exit delay". If you're running ACTSTIME in a batch

file, and you would like to pause for a few seconds to be able to

read the screen, enter a value here. The number entered is in

seconds. This delay occurs immediately after the call is

completed (telephone connection terminated). It can be any value

you wish. Entering a zero (0) means no exit delay.

Making ACTS calls on specific days of the week

ACTSTIME can be placed in a batch file that executes on a daily

basis (such as a BBS event), but only call the service on the

day(s) of the week you specify:

ACTSTIME [n]

Where "n" is the optional day(s) of the week you would like the

call to be made on. If "n" is not supplied, ACTSTIME will make

the call every time it is run. The values for "n" are 1 through

7, with 1 being Sunday, and 7 being Saturday. Here are some

examples:

ACTSTIME       Call every time executed.

ACTSTIME 1     Call only on Sunday.

ACTSTIME 26    Call on Monday & Friday.

ACTSTIME 357   Call on Tuesday, Thursday, & Saturday.

The day of the week is determined from your system's date

setting. Obviously if this is incorrect, ACTSTIME wont call on

the correct day, but once your system's clock is synchronized,

all subsequent calls should be on the proper day.

Also, I wouldn't recommend putting ACTSTIME in your AUTOEXEC.BAT

file, even with a day specified, unless used with another utility

to see that it is only executed once per day. The reason for this

is that ACTSTIME would make the call every time your system was

re-booted (or re-booted on a certain day of the week). This could

have an impact on your telephone bill if your system was re-

booted multiple times per day. I recall seeing another utility

somewhere called "DAILY" which monitors execution of programs in

batch files to see that they only get executed once per 24 hour

period.

Configuration hints, and general tips

If you leave your computer set to UTC, then line 6 should read

"00:00:00" and line 7 should be "0"

The program looks for the words "CONNECT", "BUSY", "NO DIAL

TONE", and "NO CARRIER" to control call progress. Be sure verbose

mode, and call progress words are enabled.

The program looks for the response "OK" after sending

initialization. It will retry 5 times before giving up and

terminating.

If the ACTS number is busy (or the modem sends "NO DIAL TONE" or

"NO CARRIER"), the program will wait a few seconds and redial (up

to 10 times). ACTS service has 6 incoming lines, you should

rarely, if ever, encounter a busy signal and if you do it will

probably be gone on the next call.

If ACTS doesn't answer, or connect, within 45 seconds, the

program will recycle and redial the number (up to 10 times).

If ACTS answers and somehow a synchronization cannot be made,

ACTS will automatically disconnect before 1 minute elapses. You

can never be billed for more than a 1 minute interval per call.

Distribution, Registration, BBS, General

Should you wish to compensate the author for his work, a $15 fee

is suggested. However, none is required as a requisite for use of

this program beyond 30 days. This program is shareware, and

copyrighted by Jeff Sumberg (c) 1994.

Feel free to distribute to anyone or any BBS that will take it.

This program was written by:

Jeff Sumberg

Box 212

Ringwood, NJ, 07456

I maintain a 24 hour BBS at (201)831-8152 if you have any

questions. I support all baud rates up to 14.4k at the time of

this writing.

This program was written to satisfy a need to keep my BBS and my

other computers as close to proper time as possible. All the

clocks in my machines tend to drift several minutes a month, and

if I forget to reset them, then my processing times on the BBS

are out of sync with certain events.

I am also a licensed amateur radio operator, and the times I log

for contacts need to be as close to the correct UTC as possible

for verification of contact. If a station I contact is taking 120

contacts an hour and he logs me at his end at 14:57UTC (the

correct time) and I log him here at 15:19UTC because my computer

is set wrong, my chances of getting a confirmation (QSL) card

could be lessened.

The National Institute of Standards and Technology (NIST)

NIST's time and frequency standards are where the reference for

the 60hz power frequency, television sync rate, and other

frequencies are derived. The time set into your computer from

ACTS is considered to be accurate to within 100ms (due to phone

delays, processing delays, etc). The Cesium clock that supplies

the time reference is accurate to 1 part in 100+ billion.

NIST also operates shortwave radio stations WWV in Ft Collins,

CO. and WWVH in Kauai, Hawaii. These stations transmit a double

sideband (AM) signal on very accurate carrier frequencies of 2.5,

5, 10, 15, and 20mhz. Depending on propagation (atmospheric)

conditions, at least one of these frequencies should be

receivable at any time of the day or night. WWV & WWVH also

provide atmospheric, solar, and severe storm reports at various

times during the hour. WWV uses a mans voice for time

announcements, and WWVH uses a female voice. They also speak at

different periods of the minute since in some places both

stations can be heard at the same time. WWV speaks it's ID at :00

and :30 minutes during the hour, and WWVH at :59 and :29 minutes.

The National Institute of Standards and Technology was formally

called the National Bureau of Standards (NBS).

For more information on NIST, WWV, ACTS, and the various time

services provided, write to:

National Institute of Standards and Technology

Radio Station WWV

2000 East County Road 58

Ft Collins, CO. 80524

They'll send you a very informative and interesting booklet for

free.