CD-Nap Documentation

Tomcat Softworks

15July89

CD-Nap is a hardware direct utility for Hitachi CD-ROM drives

which use the PC bus interface cards.  PC/XT/AT & PS/2 models

25/30.

The operation of these programs is similar to the Romlock

software.

CD-Nap issues a command to drives 0-3 so that if a ROM drive is

not accessed within 9 minutes, it turns off the laser and spins

down to the Stop mode.  Normally when power is on and a disc is

in the drive, the drive is always in Pause, waiting to read the

next data sector, with the disc spinning and the laser ON.

After using CD-Nap and when it is not accessed for 9 minutes, the

drive falls asleep.  The next program access which reads data or

plays audio wakes the drive up.  There will be a short delay

while the drive obtains focus on the disc and spins up to normal

speed.

Any access to the drive before 9 minutes pass, restarts the 9

minute timer.

The Nap program offers the possibility of reduced dust on the

laser lens, and increased operating life of the disc motor and

laser assembly.  On battery powered computers with internal

CD-ROM drives, CD-Nap may save power.

After Nap is used, it is effective even while playing audio.  So

if a program starts audio play and exits, after 9 minutes the

audio will stop, unless the audio software clears the Nap

command.

If CD-Nap causes your CD-ROM application program to report an

error as it tries to access the sleeping drive, please do not use

CD-Nap.

No one seems to know what improvements in disc motor or laser

life will occur by using CD-Nap.  The Hitachi drives use a

Brushless Direct Drive disc motor unlike other CD products.

If the system is left on 24 hours, or is not used for hours at a

time, CD-Nap will be effective.

Suggested use:  Execute CD-Nap from the AUTOEXEC.BAT file after

MSCDEX.EXE.  CD-Nap is cleared by Power On/Off or conditions

which also clear the lock mode on the drive.

CD-Nap is Freeware and may be copied as needed.  When you use it,

you accept any risks involved in its use or misuse.

CD-Nap was created using Quick C on a XT compatible.